- Low-cost printable metal oxides for electrocatalysis

Jaume Noguera-Gomez1, *, Juan F. Sánchez Royo1, Miguel García Tecedor2, Sixto Giménez Juliá2, Juan P. Martinez-Pastor1 and Rafael Abargues1.

1) Institut de Ciència dels Materials (ICMUV), Universitat de València, Catedrático José Beltrán 2, 46980 Paterna, Valencia, Spain.

2) Institute of Advanced Materials (INAM), Universitat Jaume I, Av. de Vicent Sos Baynat, s/n 12006, Castelló de la Plana, Spain.

*E-mail: Jaume.Noguera@uv.es

Herein, we present the recent progress in solution-processed NiOx thin films as a development of novel nanomaterials for electrocatalysis. In this work we also report our methodology to build ultrathin film layers based on sol-gel synthesis coupled by spin-coating [2] and adopted with a mild annealing process on a hot-plate (Fig 1). NiOx thin films are in-situ synthesized after deposition during a bake in a single step. The resulting film looks homogeneous without cracks nor pinholes. Our study demonstrated that NiOx films exhibit significantly enhanced electrocatalytic properties in comparison with bibliography results [3]. The most encouraging results were for NiOx films baked at low temperatures (below 150 oC) showing the best results regarding overpotential (h) for OER reaction.

We believe that our advances in solution-processed NiOx thin films baked at mild temperatures, may form the basis of a new platform for the fabrication of flexible devices with potential applications in light-driven water splitting and solar fuel cells.

Figure 1: Graphic in-situ synthesis flow-chart and performance comparison of non-stoichiometric NiOx films.

[1] Trotochaud, L. , Ranney, J. K. , Williams, K. N. , and Boettcher, S. W. Solution-cast metal oxide thin film electrocatalysts for oxygen evolution. Journal of the American Chemical Society 2012, 134, 17253–17261.

[2] Charlesbabu, J. , Gopalakrishnan, K. , Elango, M. , and Vasudevan, K. Preparation and characterization of Cd-doped ZnO thin films by spin coating method. Inorganic and Nano-Metal Chemistry 2017, 47, 1298–1303.

[3] Nadzirah, S. , Foo, K. L. , and Hashim, U. Morphological reaction on the different stabilizers of titanium dioxide nanoparticles. International Journal of Electrochemical Science 2015, 10, 5498–5512.