### ANTHROPOMETRIC DATA ANALYSIS SETS MANUAL

PREPARED BY:

Human Systems Information Analysis Center

AFRL/HEC/HSIAC

Bldg. 196

2261 Monahan Way

Wright-Patterson AFB OH  45433-7022

INTRODUCTION

In 1994 the Human Systems Information Analysis Center (HSIAC; formerly the Crew System Ergonomics Information Analysis Center [CSERIAC]) acquired a large repository of data from the Computerized Anthropometric Research and Design (CARD) Laboratory of the Air Force Research Laboratory Crew System Interface Division (formerly the Paul M. Fitts Human Engineering Division of the Armstrong Laboratory) at Wright-Patterson AFB OH.  This repository of data originally consisted of over fifty U.S. and International anthropometric surveys on both military and civilian populations.  These surveys represent more than forty-five years of research and account for hundreds of measurements on thousands of individuals.

Since these surveys were conducted by many individuals and organizations over a long period of time, the measuring techniques and the terminology were not always consistent from survey to survey.  For example, “stomach depth” in one survey may have been termed “abdominal depth” in another survey and waist circumference may have been measured at the level of the navel in one survey and the level of the subject’s waist natural indentation in the next survey.  To eliminate these inconsistencies and the confusion they create, HSIAC performed an exhaustive evaluation of all the surveys for which the documentation could be obtained, determined the similarities and differences between the surveys and the measurements, and developed a standardized coding scheme to be applied to the measurements across all of the surveys.

A total of 27 surveys have been evaluated and are now available for general use (see Page 50 for a complete listing).  Each survey contains the original ASCII dataset file and a text file that describes the survey, provides the documentation reference, and lists the specific variables that are included in the survey.  The ASCII data can be directly imported into any statistical software package on a personal computer (PC) or Macintosh computer for analysis.

This manual contains the standardized measurement definitions, a glossary of landmark definitions, and a complete list of the standardized measurements and their codes.

#### DEFINITIONS OF TRADITIONAL MEASUREMENTS

-A-

6 - Abdominal-Depth (sitting) - The depth of the abdomen.

7 - Abdominal Extension Breadth (sitting) - The horizontal breadth of the trunk at the level of the maximum protrusion of the abdomen.

8 - Abdominal-Extension (circumference) - The circumference of the lower torso at the level of the maximum anterior protrusion of the abdomen.

10 - Abdominal-Extension Depth - Subject stands erect.  The horizontal distance between the anterior point of the abdomen and the back at the same level is measured with a beam caliper.  Measure is taken at the point of maximum quiet inspiration.  The subject must not pull in his or her stomach.

11 - Abdominal-Extension -Depth-Sitting - The horizontal distance between the anterior point of the abdomen and the back at the same level is measured with a beam caliper.  The subject sits erect looking straight ahead.  The measurement is made at the maximum point of quiet respiration.

18 - Abdominal-Extension (height) - The height of the maximum anterior protrusion of the abdomen.

23 - Acromion Height - The vertical distance between a standing surface and the acromion landmark on the tip of the right shoulder is measured with an anthropometer.  The subject stands erect looking straight ahead.

25 - Acromion Height (sitting) - The vertical distance between a sitting surface and the acromion landmark on the tip of the right shoulder is measured with an anthropometer.  The subject sits erect looking straight ahead.  The shoulders and upper arms are relaxed and the forearms and hands are extended forward horizontally with the palms facing each other.

39 - Acromion-Radiale Length - .  The subject stands erect.  The distance between the acromion landmark on the tip of the right shoulder and the radiale landmark on the right elbow is measured with a beam caliper held parallel to the long axis of the arm.

42 - Acromion to Biceps Circumference Level - The surface distance along the outer edge of the arm from acromion to the biceps circumference level.

48 - Age - The age, as of the previous birthday, as reported by the subject, to which 0.5 years were added.

58 - Ankle Circumference - The minimum horizontal circumference of the right ankle is measured with a tape.

64 - Ankle Height - The height of the level of minimum circumference of the leg.

70 - Arm-Circumference Axillary - The circumference of the right upper arm perpendicular to its long axis at the level of the anterior-scye-on-the-upper arm landmark is measured with a tape.  The subject stands erect looking straight ahead with shoulders and upper extremities relaxed and the palms facing the sides.

72 - Arm Circumference -Biceps (Relaxed) - The maximum circumference of the right arm at the level of the biceps measured with the arm hanging relaxed.

73 - Arm Circumference Biceps (Relaxed- left) - The maximum circumference of the left arm at the level of the biceps measured with the arm hanging relaxed

77 - Arm Circumference - Biceps (Flexed) - The circumference of the right upper arm around the flexed biceps muscle is measured with a tape held perpendicular to the long axis of the upper arm.  The subject stands with the upper arm extended horizontally and the elbow flexed 90.  The fist is clenched and held facing the head, and the subject exerts maximum effort in “making a muscle”.

78 - Arm Circumference - Biceps (Flexed - left) - The circumference of the left upper arm around the flexed biceps muscle is measured with a tape held perpendicular to the long axis of the upper arm.  The subject stands with the upper arm extended horizontally and the elbow flexed 90.  The fist is clenched and held facing the head, and the subject exerts maximum effort in “making a muscle”.

80 - Arm Reach From Wall - The distance from the wall to the tip of the middle finger measured with the subject’s shoulders against the wall, the hand and arm extended forward.

81 - Arm Reach From Wall - Sitting- Subject’s right arm and fingers are extended forward parallel to the floor.  Measure the distance from the back of the right shoulder to the tip of the middle finger.

90 - Axilla Height - The vertical distance between a standing surface and the right axillary fold, as designated by the anterior-scye-on-the-torso landmark, is measured with an anthropometer.  The subject stands erect looking straight ahead.

91 - Axilla to Waist Length - The surface distance from the armpit to waist level.

-B-

92 - Back Curvature - The tape distance across the back as measured from the point on the right side in the midaxillary plane at chest level to the similar left point.

93 - Back Arc at Bust - The surface distance across the back between the midaxillary lines at the level of the bra points (or chest level for men).

95 - Back Arc at Waist - The surface distance across the back between the midaxillary lines at waist level.

103 - Biacromial Breadth - The distance between the right and left acromion landmarks at the tips of the shoulders is measured with a beam caliper.  The subject sits erect.  The shoulders and upper arms are relaxed and the forearms and hands are extended forward horizontally with the palms facing each other.

110 - Biceps Skinfold - A vertical fold of skin is measured at the anterior mid point of the contracted bicep muscle.  This midpoint is found by measuring the distance between the lateral epicondyle of the left humerus and the acromial landmark.

122 - Bideltoid Breadth - The maximum horizontal distance between the lateral margins of the upper arms on the deltoid muscles is measured with a beam caliper.  The subject sits erect looking straight ahead.  The shoulders and upper arms are relaxed and the forearms and hands are extended forward horizontally with the palms facing each other.

126 - Bigonial Breadth - The breadth of the jaw across the gonial angles.

130 - Biiliocristale Breadth - The breadth of the torso measured between the superior points of the ilia in the midaxillary plane.

134 - Bimalleolar Breadth - The horizontal distance between the maximum protrusions of the ankle bones lateral and medial malleoli) of the right foot is measured with a Holtain caliper.  The subject stands with the weight equally distributed on both feet.

138 - Biocular Breadth - The distance between the outer corners of the eyes.

140 - Bispinous Breadth - The straight-line distance between the right and left anterior superior iliac spine landmarks is measured with a beam caliper.  The subject stands looking straight ahead with the heels together and the weight distributed equally on both feet.

142 - Bitragion Breadth - The breadth of the head as measured from the right to left tragion.

144 - Bitragion - Coronal Arc - The surface distance between the right and left tragion landmarks across the top of the head is measured with a tape.  The head is in the Frankfort plane.

146 - Bitragion - Crinion Arc - The surface distance between the right and left tragion landmarks across the top of the forehead at the lowest point of the hairline (crinion).  Measured with a tape.

150 - Bitragion - Menton (chin) Arc - The surface distance between the right and left tragion landmarks across the chin landmark at the tip of the chin is measured with a tape.  The teeth are lightly occluded.

152 - Bitragion - Minimum Frontal Arc - The surface distance between the right and left tragion landmarks across the forehead just above the ridges of the eyebrow is measured with a tape.

153 - Bitragion Posterior Arc - The distance from right to left tragion measured with the tape passing over the base of the skull.

156 - Bitragion Submandibular Arc - The surface distance between the right and left tragion landmarks across the submandibular landmark at the juncture of the jaw and the neck is measured with a tape.  The head is in the Frankfort plane and the teeth are lightly occluded.

158 - Bitragion-Subnasale Arc - The surface distance between the right and left tragion landmarks across the subnasale landmark just under the nose is measured with a tape.

161 - Bitrochanterion Breadth - The horizontal distance between the trochanters measured with the flesh compressed.

165 - Bizygomatic Breadth - The maximum horizontal breadth of the face (between the zygomatic arches) is measured with a spreading caliper.

167 - Body Length-Supine-Knees Flexed - The distance from the top of the head to the tip of the toes when the subject lies supine, knees raised so the angle between the upper and lower legs approximates 60 degrees and the toes are touching a wall.

169 - Bowed Torso Height - The distance from the floor to the top of the head measured as the subject stands with the feet 12 inches apart, palms of the hands on the kneecaps and the head as close to the Frankfort Plane as possible.

172 - Bustpoint to Bustpoint Breadth - The distance between the right and left bustpoint landmarks on women and the center of the nipples (thelion) on men is measured with a beam caliper.  The subject stands erect looking straight ahead.  The shoulders an upper extremities are relaxed.

178 - Buttock Circumference - The circumference of the body measured at the level of the maximum posterior protuberance of the buttocks.

179 - Buttock Circumference - Sitting - The circumference of the buttocks measured with the tape passing under the buttocks and brought up diagonally across the lap.

185 - Buttock Curvature - The surface distance across the back between the midaxillary lines at the level of the maximum protrusion of the buttocks.

186 - Buttock Depth - The depth of the torso at the level of the maximum posterior protrusion of the buttock.

187 - Buttock-Heel Length - The distance from the base of the heel to a wall against which the subject sits erect with the legs maximally extended forward along the sitting surface.

188 - Buttock Height - The vertical distance between a standing surface and the level of the maximum protrusion of the right buttock is measured with an anthropometer at the right side of the thigh.  The subject stands erect with the heels together.

194 - Buttock-Knee Length - The horizontal distance between a buttock plate placed at the most posterior point on either buttock and the anterior point of the right knee is measured with an anthropometer.  The subject sits erect.  The thighs are parallel and the knees flexed 90 with the feet in line with the thighs.

200 - Buttock-Popliteal Length - The horizontal distance between a buttock plate placed at the most posterior point on either buttock and the back of the right is measured with an anthropometer.  The subject sits erect.  The thighs are parallel and the knees flexed 90 with the feet in line with the thighs.

-C-

207 - Calf Circumference - The maximum horizontal circumference of the right calf is measured with a tape.  The subject stands erect with the heels approximately 10 cm  apart and the weight distributed equally on both feet.

209 - Calf Circumference - left-previous measurement repeated on opposite leg

215 - Calf Height - The vertical distance between a standing surface and the level of the maximum circumference of the right calf is measured with an anthropometer.  The subject stands erect with the heels together and the weight distributed equally on both feet.

219 - Cervicale Height - The vertical distance between a standing surface and the cervicale landmark on the spine at the base of the neck is measured with an anthropometer.  The subject stands erect with the head in the Frankfort plane.

220 - Cervicale Height (Sitting) -The vertical distance between a sitting surface and the cervicale landmark on the spine at the base off the neck is measured with an anthropometer.  The subject sits erect looking straight ahead.  The shoulders and upper arms are relaxed and the forearms and hands are extended forward horizontally with the palms facing each other.  The thighs are parallel and the knees are flexed 90.

223 - Chest Breadth - The maximum horizontal breadth of the chest at the level of the right bustpoint on women or the nipple on men is measured with a beam caliper.  The subject stands erect looking straight ahead with the heels together, the weight distributed equally on both feet.  The measurement is taken at the maximum point of quiet respiration.

227 - Chest Breadth (bone) - The breadth of the torso at nipple level measured with pressure.

230 - Chest Circumference - The maximum horizontal circumference of the chest at the fullest part of the breast is measured with a tape.  The subject stands erect looking straight ahead.  The shoulders and upper extremities are relaxed.

231 - Chest Circumference at Scye - The horizontal circumference of the chest at the level of the scye at midspace landmark is measured with a tape.  The subject stands erect looking straight ahead.  The shoulders and upper extremities are relaxed.

232 - Chest circumference (below bust) -The horizontal circumference of the chest at the level of the inferior juncture of the lowest breast with the rib cage is measured with a tape.  On women, the tape may lie on the bra.  The subject stands erect looking straight ahead.  The shoulders and upper extremities are relaxed.

233 - Chest Circ - Inspiration - Subject stands with arms slightly raised.  Holding the tape in a horizontal position immediately below the breasts, measure the circumference of the chest at maximum inhalation.

235 - Chest Depth - Sitting -  The subject sit erect with his or her back against a wall.  At the level of the nipple, the measurement is the horizontal distance between the most anterior point of the chest and the wall.

236 - Chest Depth - The horizontal distance between the chest, at the level of the right bustpoint on women or the nipple on men, and the back at the same level is measured with a beam caliper.  The subject stands erect looking straight ahead.  The shoulders and upper extremities are relaxed.

237 - Chest Height - The vertical distance between a standing surface and the right bustpoint on women or the nipple on men is measured with an anthropometer.  The subject stands erect looking straight ahead.  The heels are together with the weight distributed equally on both feet.  The shoulders and upper extremities are relaxed.

249 - Crotch Height - The vertical distance between the standing surface and the crotch is measured with an anthropometer.  The subject stands erect looking straight ahead.  The heels are together and the weight is distributed equally on both feet.

251 - Crotch Length (Natural Waist Indent) - The distance between the abdomen at the level of the natural indentation of the waist to the same level on the back is measured with a tape passing through the crotch to the right of the genitalia.  The tape is held vertically both in front and in back.  The subject stands erect looking straight ahead.

252 - Crotch Length (Umbilicus) - The surface distance from omphalion to the waist level on the right side of the subject’s back measured through the crotch and over the middle of the buttock.

253 - Crotch Length (Posterior Natural) - The surface distance from the crotch at the inner thigh landmark to the back of the waist at the posterior natural-indentation landmark is measured with a tape.  The tape passes between the buttocks to the back of the waist.  The subject stands with the left foot on a platform so that the knee is flexed.

254 - Crotch Length (Posterior Omphalion) - The surface distance from the crotch at the inner thigh landmark to the back of the waist at the level of the center of the navel (omphalion) is measured with a tape.  The tape passes between the buttocks to the back of the waist.  The subject stands with the left foot on a platform so that the knee is flexed

-D-

265 - Dactylion Height - The height of the tip of the middle finger measured with the arm, hand, and finger extended downward.

269 - Deltoid Arc - The surface distance from acromion to the point, just below the deltoid, where the muscle disappears from view.

-E-

277 - Ear Breadth - The maximum breadth of the right ear perpendicular to its long axis is measured with a sliding caliper.

280 - Ear Length - The length of the right ear from its highest to lowest points on a line parallel to the long axis of the ear is measured with a sliding caliper.

282 - Ear Length - Above Tragion - The distance from the right tragion landmark to the top of the right ear on a line parallel to the long axis of the ear is measured with a sliding caliper.

285 - Ear Protrusion - The horizontal distance between the mastoid process (the bony area behind the bottom of the ear) and the outside edge of the right ear at its most lateral point is measured using a sliding caliper with its slide reversed.

286 - Ear-to-Ear Breadth - The horizontal distance between the most lateral extensions of the ears.

293 - Elbow Breadth (bone) - The distance between the medial and lateral epicondyles of the humerus measured with the flesh compressed.

297 - Elbow Breadth (bone - left) - Previous measurement repeated on opposite arm.

301 - Elbow Circ-Fully Bent - The distance around the maximum prominence of the olecranon and the inside face of the elbow measured with the elbow fully flexed and the fingers touching the shoulder.

303 - Elbow Circumference (extended) - The circumference of the right elbow in a plane perpendicular to the long axis of the arm is measured with a tape passing around the elbow at the level of the olecranon-center landmark.  The subject stands with the arm straight and slightly away from the side.

305 - Elbow Circumference (flexed) - The circumference of the elbow measured over its tip and through its crotch with the arm bent 90, the upper arm horizontal, and the fist clenched.

307 - Elbow-Grip Length - The distance from the tip of the bent elbow to the center of the clenched fist.

309 - Elbow Height - The height of the radiale.

312 - Elbow-Rest Height - The vertical distance between a sitting surface the olecranon landmark on the bottom of the flexed right elbow is measured with an anthropometer.  The subject sits erect looking straight ahead.  The shoulders and upper arms are relaxed and the forearms and hands are extended forward horizontally with the palms facing each other.

318 - Elbow-Elbow Breadth - The distance across the lateral surfaces of the elbows measured with the elbows flexed and resting lightly against the body.

321- Elbow Functional Reach -  The subject stands erect with his or her back against a wall.  The left upper arm is held horizontal and parallel to the floor with the olecranon process in light contact with the wall.  The hand is positioned such that the thumb is held parallel to the forearm and the fore-finger is held in opposition to the thumb.  The distance between the tips of the these two fingers and the wall is recorded.

324 - Elbow-Wrist Length - The distance from the tip of the elbow to the tip of the styloid process of the radius.

325 - Elbow Span -  The subject stands erect with his or her back against a wall.  The arms are stretched horizontally  and the forearms are held in 90 of flexion parallel with the end wall and floor.  The distance between the right and left olecranon is recorded.

326 - External-Canthus (to Occiput) - The distance from the back of the head to the outer corner of the right eye.

327 - External-Canthus (to Vertex) - The vertical distance from the outer corner of the eye (external canthus) to the level of the top of the head.

328 - External Canthus (to Wall) - The horizontal distance from the outer corner of the eye (external canthus) to the to the coronal plane tangent to the back of the head

329 - Eye Height-Standing - The height of the inner corner of the eye.

330 - Eye Height (sitting) - The vertical distance between a sitting surface and the ectocanthus landmark on the outer corner of the right eye.

-F-

340 - Fibula Height - The height of the superior tip of the fibula.

344 - Finger III Diameter - The diameter in the middle finger as determined by the smallest hole in which the finger can be inserted.

348 - First Phalanx III Length - With the subject’s right hand held in a fist, measure the length of the first segment of the middle finger across the surfaces of the third metacarpal and the second phalanx.

352 - Fist Circumference - The circumference of the clenched fist (the thumb lying across the end of the fist) measured with the tape passing over the thumb and the knuckles.

356 - Foot breadth - The maximum breadth of the right foot is measured on a footbox scale.  The subject stands with each foot in a footbox and the weight distributed equally on both feet.

358 - Foot Circumference (Ball of Foot) - The circumference of the foot at the first and fifth metatarsophalangeal landmarks on the ball of the right foot is measured with a tape.  The subject stands with the feet about 10 cm apart and the weight distributed equally on both feet.

362 - Foot Length - The length of the foot measured parallel to its long axis.

370 - Forearm Circumference (Extended) - The maximum circumference of the lower arm.

371 - Forearm Circumference (Flexed) - The subject stands with the upper arm extended forward horizontally, the elbow flexed 90, and the fist tightly clenched and held facing the head.  The circumference of the flexed right forearm is measured with a tape passing across the crease at the juncture between the upper arm and the forearm.  The measurement is made in a plane perpendicular to the long axis of the forearm.

375 - Forearm-Hand Length - The subject stands erect with the upper arms hanging at the sides and the right elbow flexed 90.  The hand is held out straight with the palm facing inward.  The horizontal distance between the back of the tip of the right elbow to the tip of the right middle finger is measured with a beam caliper.

378 - Forearm-Forearm Breadth - The maximum horizontal distance across the upper body between the out sides of the forearms is measured with a beam caliper.  The subject sits erect looking straight ahead.  The shoulders and upper arms are relaxed and the forearms and hands are extended forward horizontally with the palms facing each other.

386 - Functional Leg Length - Sitting erect on a chair, the subject extends his or her right leg forward with the knee straightened.  The distance from the heel of the foot to the posterior waist landmark is measured along the long axis of the leg.

-G-

391 - Glabella to Occiput - The distance from the most anterior point of the forehead between the brow ridges to the back of the head.

393 - Glabella to Top of Head (Vertex) - The distance from the most anterior point of the forehead between the brow ridges to the level of the top of the head.

394 - Glabella to Wall - The distance from the most anterior point of the forehead between the brow ridges to wall.

395 - Gluteal Arc - The surface distance over the buttock from the gluteal furrow to waist level.

398 - Gluteal-Furrow Height - The vertical distance between a standing surface and the lowest point of the gluteal furrow(s) under the right buttock is measured with an anthropometer.  The subject stands erect with the heels together and the weight distributed equally on both feet.

402 - Grip Diameter (Inside) - Subject holds a cone around the largest circumference that can be grasped with the thumb and middle finger just touching.  Record the diameter of the cone corresponding to this maximum circumference.

404 - Grip Diameter (Outside) - Subject holds a cone at the largest circumference that can be grasped with the thumb and middle finger just touching.  With a sliding caliper, measure from the joint of the first and second phalanges of the thumb to the knuckle of the middle finger.

405 - Grip-Reach Overhead - The height of a pointer held horizontal in the subject’s fist when the arm is maximally extended upward.

407 - Grip Strength - Strength as measured on a Smeadley dynamometer.

-H-

411 - Hand Breadth at Metacarpale - The breadth of the right hand between the landmarks at metacarpale II and metacarpale V is measured with a sliding caliper.  The subject places the palm on a table, the fingers together and the thumb abducted.  The middle finger is parallel to the long axis of the forearm.

413 - Hand Breadth at Thumb - The breadth of the hand as measured across the distance of the metacarpal bones.

416 - Hand Circumference at Metacarpale - The hand circumference of the right hand is measured with a tape passing over the landmarks at metacarpale II and metacarpale V.  The subject places the palm on a table, the fingers together, and the thumb abducted.  The middle finger is parallel to the long axis of the forearm.

417 - Hand Circumference over Thumb - The circumference of the hand measured in a plane at right angle to the long axis of the hand, with the tape passing over the proximal joint of the thumb.

420 - Hand Length - The length of the right hand between the stylion landmark on the wrist and the tip of the middle finger is measured with a Poech sliding caliper.  The subject places the palm on a table with the fingers together, and the thumb abducted.  The middle finger is parallel to the long axis of the forearm.

424 - Hand Skinfold - The thickness of a skinfold picked up at the middle of the back of the right hand and following the long axis of the hand.

425 - Hand Thickness at Metacarpale III - The thickness of the metacarpal-phalangeal joint of the middle finger.

427 - Head Breadth - The maximum horizontal breadth of the head above the attachment of the ears is measured with a spreading caliper.

430 - Head Circumference - The maximum circumference of the head above the attachment of the ears is measured with a tape passing just above the ridges of the eyebrows and around the back of the head.

433 - Head Diagonal (Inion to Pronasale) - The transverse distance from the tip of the nose to the inion.

435 - Head Diagonal (Maximum from Menton) - The maximum transverse distance from the chin (jaw closed) and the vertex (top of head).

436 - Head Diagonal (Maximum from Nuchale) - The maximum transverse distance from the lowest point palpable at the base of the skull to the tip of the nose.

437 - Head Diagonal (Maximum from Menton to Occiput) - The maximum transverse distance from the tip of the chin to the back of the head.

441 - Head Length - The distance from the glabella landmark between the browridges to the posterior point on the back of the head.  Measured with a spreading caliper.

445 - Heel Ankle Circumference - The diagonal circumference of the foot measured with the tape passing under the tip of the heel and over the instep at the foot-leg junction..

450 - Heel Breadth - The maximum horizontal distance between the medial an lateral points on the inside and outside of the right heel, at or posterior to the lateral malleolus landmark, is measured with a Holtain caliper.

457 - Hip Breadth - The horizontal distance between the lateral buttock landmarks on the sides of the hips is measured with a beam caliper.  The subject stands erect with the heels together and the weight distributed equally on both feet.

459 - Hip Breadth (Sitting) - The distance between the lateral points of the hips or thighs (whichever are broader) is measured with a beam caliper.  The subject sits erect with the feet and knees together.

465 - Hip Circumference at Trochanterion - The circumference of the torso measured at the level of the trochanterions.

466 - Hip Circumference (7 below waist) - The circumference of the lower torso measured at a level 7” below waist level.

468 - Hip Circumference (9 below waist) - The circumference of the lower torso measured at a level 9” below waist level.

-I-

489 - Iliocristale Height - The subject stands erect.  The vertical distance between a standing surface and the iliocristale landmark on the top of the right side of the pelvis is measured with an anthropometer.

490 - Iliospinale Height - Subject stands erect.  The vertical distance from the floor to the left iliospinale is recorded.

493 - Instep Circumference - The circumference of the foot in a plane perpendicular to the long axis of the foot at the level of the metatarsal cuneiform joint.

496 - Instep Length (Ball of Foot) - The distance, measured parallel to the long axis of the foot, from the level of the heel to the point of maximum medial protuberance of the foot.

500 - Interocular Distance - The distance between the inner corners of the eyes.

503 - Interpupillary Distance - The distance between the two pupils is measured with a pupilometer.

506 - Interscye Distance - Subject stands erect looking straight ahead.  The distance across the back between the right and left posterior axillary fold landmarks.  Measured with a tape.  Tape is held on the skin surface except where the tape spans the hollow of the back.  Shoulders and upper extremities are relaxed.

507 - Interscye Distance - Maximum - The tape distance across the back between the scye points measured with the subject sitting erect with his arms extended forward and horizontal.

508 - Interscye Distance - Mid - Subject stands erect looking straight ahead.  The distance across the back between the right and left midscye landmarks is measure with a tape.  The tape is held on the skin surface except where it spans the hollow of the back.  Shoulders and upper extremities are relaxed.

510 - Interscye - Front - The surface distance across the front of the torso between points midway between the anterior edges of the armpit and the acromiale points.

-J-

511 - Juxtanipple Skinfold - The thickness of a skinfold picked up just superior to the nipple and parallel to the lateral margin of the pectoral muscle.

-K-

512 - Knee Breadth (Bone) - Subject stands erect.  The transverse distance between the lateral and medial femoral epicondyles of the right leg is measured with a sliding caliper.

513 - Knee Breadth (Bone - left) - Subject stands erect.  The transverse distance between the lateral and medial femoral epicondyles of the left leg is measured with a sliding caliper.

515 - Knee Circumference - The horizontal circumference of the right knee at the level of the midpatella landmark at the center of the knee.  Measured with a tape.  Subject stands erect with the feet about 10 cm apart with weight distributed equally on both feet.

516 - Knee Circumference - Fully Bent - The circumference around the maximum knee prominence and through the crease behind the knee, measured with the subject in a squatting position with the knee fully flexed.

517 - Knee Circumference - Sitting - The maximum circumference of the right knee as measured with a tape passing over the popliteal area brought up at 45 over the knee.

528 - Knee Height - Flexed, supine - The height of the highest point on the knee when the subject lies supine, knees raised so that the angle between the upper and lower legs approximates 60 degrees and the toes are touching a wall.

529 - Knee Height - Sitting - The vertical distance between a footrest surface and the suprapatella landmark at the top of the right knee.  Measured with an anthropometer.  Subject sits with the thighs parallel, knees flexed to 90 and the feet in line with the thighs.

536 - Knee to Knee Breadth - The maximum horizontal distance across the lateral surfaces of the knees measured with the knees gently touching.

538 - Kneeling Height - The subject kneels with his or her knees and feet together with fists clenched and on the floor in front of his or her knees.  The arms are vertically positioned and the head is in line with the long axis of the body.  The vertical distance from the floor to the highest point on the head is recorded.

540 - Kneeling Leg Length - The subject kneels with the knees and feet together with fists clenched and on the floor in front of his or her knees.  The distance from the most rearward point on the foot to the most forward point on the head is recorded.

-L-

542 - Lateral Femoral Epicondyle Height - The vertical distance between a standing surface and the lateral femoral epicondyle landmark on the outside of the right knee is measured with an anthropometer.  Subject stands erect with the heels together and the weight distributed equally on both feet.

543 - Lateral Malleolus Height - The vertical distance between a standing surface and the lateral malleolus landmark on the outside of the right ankle.  Measured with a modified sliding caliper.  Subject stands erect with the heels together and the weight distributed equally on both feet.

547 - Lip Length - The distance between the corners of the mouth is measured while the facial muscles are relaxed.

549 - Lip Length - Smiling - The distance between the corners of the mouth is measured while the subject smiles broadly.

552 - Lip Protrusion to Occiput - The distance from the most anterior point of the lips to the wall (or back of head).

553 - Lip Protrusion to Wall - The horizontal distance from the most anterior point of the lips to the coronal plane tangent to the back of the head.

555 - Lip to Lip Length - The distance, in the midsagittal plane, from the lower margin of the lower lip to the upper margin of the upper lip.

-M-

569 - Max Frontal Breadth - Forehead breadth as measured between the maximum bulges of the brow ridges at about the ends of the eyebrow.

572 - Max Reach From Wall - The distance from the wall to the tip of the middle finger measured with the subject’s back and left shoulder pressed against the wall, the right shoulder thrust as far forward possible and the arm and hand extended horizontally.

576 - Medial Calf Skinfold - The thickness of a skinfold on the medial surface of the calf at the level of the maximum circumference picked parallel to the long axis of the lower leg.  Measured with the knee flexed about 90.

579 - Medial Malleolus Height - The height of the most medially projected point of the medial ankle bone.

583 - Menton-Crinion Length - The distance from the bottom surface of the chin to the midpoint of the hairline.  Not measured on bald and balding.

585 - Menton Projection - The Distance from the most forward point of the chin to the juncture of the neck and the bottom of the jaw.

586 - Menton-Nasal Root Depression (Sellion) - The distance between the menton landmark at the bottom of the chin and the sellion landmark at the deepest point of the nasal root depression.  Measured with a sliding caliper.  The teeth are lightly occluded.

588 - Menton to Occiput - The distance from the tip of the chin to the back of the head.

592 - Menton to Subnasale Length - The distance from the lower edge of the tip of the chin to the base of the nasal septum.

595 - Menton to Top of Head (Vertex) - The distance from the lower edge of the tip of the chin to the level of the top of the head.

596 - Menton to Wall - The distance from the tip of the chin to the wall.

601 - Metacarpale Height - The height of the knuckle where the middle finger joins the palm.

605 - Midaxillary Line at Umbilicus Level Skinfold - The thickness of a skinfold picked up at xiphoid level in the midaxillary line.

612 - Midshoulder Height (Sitting) - The vertical distance between a sitting surface and the midshoulder landmark at the top of the right shoulder is measured with an anthropometer.  Subject sits erect looking straight ahead with the shoulders and upper arms relaxed. The forearms and hands are extended forward horizontally with the palms facing each other.

615 - Mid-Thigh Circumference - Subject stands erect.  Measure is taken horizontally midway between the crotch and knees.  Left side only.

616 - Minimum Frontal Arc - The minimum distance across the temporal crest between their points of maximum indentation.

618 - Minimum Frontal Breadth - The breadth of the forehead across the lateral bony ends of the brow ridges.

-N-

625 - Nasal Root Breadth - The breadth of the nasal root.

629 - Nasal Root Height - The height of the deepest point in the nasal root depression.

631 - Nasal Root Depression to Occiput - The distance from the deepest point in the nasal root depression to the wall (or back of head)

633 - Nasal Root Depression to Vertex - The distance from the deepest point in the nasal root depression (sellion) to the level of the top of the head.

634 - Nasal Root Depression to Wall - The horizontal distance from the nasal root depression to the coronal plane tangent to the back of the head.

636 - Neck Circumference (Base) - The circumference of the base of the neck is measured by a tape passing over the drawn lateral and anterior neck landmarks.  The subject stands erect with the head in the Frankfort plane.  Shoulders and upper extremities are relaxed.

637 - Neck Circumference (Below Larynx) -  The maximum circumference of the neck at a point just below the bulge of the thyroid cartilage (Adam’s apple).  Measured with a tape.

639 - Neck Circumference (Over Larynx) - The circumference of the neck at the level of the infrathyroid landmark (Adam’s apple).  Measured with a tape.

641 - Neck Height (Lateral) - The vertical distance between a standing surface and the trapezius landmark on the right side of the neck.  Measured with an anthropometer.  Subject stands erect with the head in the Frankfort plane.  Shoulders and upper extremities are relaxed.

644 - Neck Length (Anterior) -The surface distance from suprasternale to the junction, in the midsagittal plane, of the jaw and neck.

647 - Neck Length (Posterior - Nuchale) - The surface distance from the nuchale (the lowest palpable point of the surface distance along the spine from cervicale to the occiput) to the cervicale.

648 - Neck - Bustpoint (Thelion) Length - The distance between the trapezius landmark at the right side of the neck and the right bustpoint landmark on women or the right nipple (thelion) on men.  Measured with a tape.  Subject stands erect looking straight ahead.  Shoulders and upper extremities are relaxed.

649 - Nose Length - The distance from the nasal root depression (sellion) to the bottom of the nose.

651 - Nose breadth - Maximum breadth of the nose

652 - Nose Protrusion - The maximum anterior protrusion of the nose.

-O-

653 - Overhead Reach - The vertical distance between a standing surface and the tip of the right middle finger when the arm is extended overhead.  Subject stands facing a wall-mounted scale with both arms extended overhead parallel to each other.  The toes are 20 cm from the wall and the feet are about 10 cm apart.  The palms of the hands rest on the wall.

654 - Overhead Reach - Sitting - The height of the tip of the middle finger above the sitting surface measured with the subject sitting erect, right side against a wall, left hand in lap, and the right arm and hand extended upward with the palm against the wall.

655 - Overhead Reach - Forward - Subject stands with heels 23 centimeters apart and toes 15 centimeters from the wall.  Arms are extended overhead, fists touching and against the wall, with the first phalanges horizontal.  Mark the highest point on the first phalanges on the wall and record the height of the mark.

656 - Overhead Reach - Extended - The vertical distance between a standing surface and the tip of the right middle finger when the arm is extended overhead as high as possible.  The subject stands on his/her toes facing a wall-mounted scale with both arms parallel and extended overhead as high as possible.  The toes are 20 cm from the wall and the feet are about 10 cm apart.  Palms of the hands rest on the wall.

-P-

657 - Palm Length - The distance from the base of the hand to the furrow where the middle finger folds upon the palm.

663 - Patella Bottom Height - The height of the bottom edge of the kneecap.

666 - Patella Top Height - The height of the top edge of the kneecap

670 - Penale Height - The height of the upper edge of the junction of the penis with the abdomen.

674 - Philtrum Length - The distance from the edge of the upper membranous lip to the base of the nasal septum.

678 - Popliteal Height - Sitting - The vertical distance from a footrest surface to the back of the right knee.  Measured with an anthropometer.  The subject sits with the thighs parallel, the feet in line with the thighs, and the knees flexed 90.

690 - Pronasale to Occiput - The distance from the tip of the nose to the back of the head.

692 - Pronasale to Vertex (Top of Head) - The distance from the tip of the nose to the level of the top of the head.

694 - Pronasale to Wall - The distance from the tip of the nose to the wall.

-R-

698 - Radiale-Stylion Length - The distance between the radiale landmark on the right elbow and the stylion landmark on the right wrist.  Measured with a beam caliper held parallel to the long axis of the forearm.  Subject stands with the arms relaxed at the sides.  The hand an fingers are held straight in line with the long axis of the forearm.

-S-

705 - Sagittal-Arc (Nuchale) - The distance over the top of the head from the glabella (between the brow ridges) to nuchale at the base of the skull.

732 - Scye Circumference over Acromion - The vertical circumference of the right upper arm (scye).  Measured with a tape passing through the armpit and over the acromion landmark on the tip of the shoulder.  The subject stands erect looking straight ahead.  The shoulders and upper extremities are relaxed with the palms facing the thighs.

737 - Scye depth - The surface distance along the spine between the cervicale landmark on the base of the back of the neck and the scye-level-at-midspine landmark is measured with a tape.  The subject stands erect with the head in the Frankfort Plane.  The shoulders and upper extremities are relaxed.

743 - Shoulder Breadth - Bowed - The maximum breadth of the shoulders measured as the subject stands, feet 12 inches apart, the palms of the hands on the kneecaps, and the head as close to the Frankfort Plane as possible.

745 - Shoulder Breadth - Reaching Overhead - Subject stands with heels 23 centimeters apart and toes 15 centimeters from the Wall.  Arms are extended overhead, fists together and against the wall with the first phalanges horizontal.  Measure the maximum horizontal distance across the arms or shoulders, whichever is widest.

747 - Shoulder Circumference - The horizontal circumference of the shoulders at the level of the maximum protrusion of the right deltoid muscle is measured with a tape.  Subject stands erect looking straight ahead.  Shoulders and upper extremities are relaxed with the palms facing the thighs.

751 - Shoulder-Elbow Length - The distance between the acromion landmark on the tip of the right shoulder and the olecranon landmark on the bottom of the right elbow.  Measured with a beam caliper parallel to the long axis of the upper arm.  Subject stands with the right upper arm hanging at the side and the elbow flexed 90.  The hand is straight and the palm faces inward.

754 - Shoulder Length - The surface distance between the trapezius landmark at the base of the side of the neck and the acromion landmark on the tip of the right shoulder.  Measured with a tape.  The subject stands looking straight ahead.  The shoulders and upper extremities are relaxed.

758 - Sitting Height - The vertical distance between a sitting surface and the tip of the head.  Measured with an anthropometer.  The subject sits erect with the head in the Frankfort plane.  The shoulders and upper arms are relaxed and the forearms and hands are extended forward horizontally with the palms facing each other.  The thighs are parallel and the knees are flexed 90 with the feet in line with the thighs.

760 - Sitting Height - Relaxed - The vertical distance from the sitting surface to the top of the head is measured with the subject sitting relaxed.

764 - Sleeve-Inseam Length - The distance from the anterior edge of the armpit to the base of the thumb.

797 - Sleeve - Outseam Length - The straight line distance between the acromion landmark on the tip of the right shoulder and the stylion landmark on the right wrist.  Measured with a tape.  Subject stands erect with both arms straight at the side and the palms facing forward.

799 - Span - The distance between the tips of the right and left middle fingers when the subject’s arms are maximally extended laterally.

801 - Sphyrion Height - The height of sphyrion.

802 - Spine to Elbow Length (SL) - The horizontal surface distance between the midspine landmark and the olecranon-center landmark on the tip of the raised elbow is measured with a tape.  The subject holds his/her arms up in a horizontal position parallel to the standing surface and joins them by bringing the fists together at the metacarpophalangeal and proximal interphalangeal knuckles.  The forearms and fists are in a straight line.

803 - Spine to Scye Length (SL) - The horizontal surface distance between the midspine landmark and the right posterior-diagonal-scye landmark at the back of the raised right arm near the armpit is measured with a tape.  The subject holds his/her arms up in a horizontal position parallel to the standing surface and joins them by bringing the fists together at the metacarpophalangeal and proximal interphalangeal knuckles.  The forearms and fists are in a straight line.

804 - Spine to Wrist Length (SL) - The horizontal surface distance from the midspine landmark, across the olecranon-center landmark at the tip of the raised right elbow, to the dorsal wrist landmark is measured with a tape.  The subject holds his/her arms up in a horizontal position parallel to the standing surface and joins them by bringing the fists together at the metacarpophalangeal and proximal interphalangeal knuckles.  The forearms and fists are in a straight line.

805 - Stature - The vertical distance from a standing surface to the top of the head.  Measured with an anthropometer.  The subject stands erect with the head in the Frankfort plane.  The heels are together with the weight distributed equally on both feet.  The shoulders and upper extremities are relaxed.  The measurement is taken at the maximum point of quiet respiration.

806 - Stature (Clothed) - The distance from the floor to the top of the head measured with the subject wearing normal indoor apparel.

807 - Stature (Maximum) - The height of the top of the head measured when the subject has, after taking a deep breath, maximally extended himself vertically while keeping his feet flat on the floor.

808 - Stature (Reported) - The height as given by the subject immediately prior to being measured.

814 - Stomion-to-Occiput - The distance from the point of contact of the lips in the midsagittal plane to the back of the head.

815 - Stomion to Top of Head (Vertex) - The distance from the point of contact of the lips in the midsagittal plane to the level of the top of the head.

816 - Stomion to Wall - The horizontal distance from the wall to the stomion (the point of contact of the lips in the midsagittal plane).

821 - Strap Length - The surface distance from the right bustpoint landmark on women or the right nipple is measured with a tape passing over the left and right lateral neck landmarks.  The subject stands erect with the head in the Frankfort plane.  The shoulders and upper extremities are relaxed.

825 - Subnasale to Nasal Root Depression (Sellion) - The distance from the base of the nasal septum to the plane of the top of the head.

829 - Subnasale to Occiput - The distance from the base of the nasal septum to the back of the head

830 - Subnasale to Top of Head (Vertex) - The distance from the base of the nasal septum to the plane of the top of the head.

831 - Subnasale to Wall - The distance from the base of the nasal septum to the wall.

833 - Subscapular Skinfold - The thickness of a skinfold picked up just below the inferior angle of the right scapula and parallel to the tension lines of the skin.

834 - Subscapular Skinfold II - The previous measurement repeated on the opposite side.

837 - Substernale Height - The height of the midpoint of the lower edge of the breast bone.

841 - Suprasternale Height - The vertical distance between a standing surface and the suprasternale landmark on the notch at the top of the breastbone is measured with an anthropometer.  Subject stands erect looking straight ahead.  The shoulders and upper extremities are relaxed.

844 - Suprailiac Skinfold - The thickness of a skinfold picked up in the midaxillary line at the level of the crest of the ilium.

845 - Suprailiac Skinfold II - The previous measurement repeated on the opposite side.

848 - Suprapatella Skinfold - The thickness of a skinfold picked up on the lower thigh above the patella.

-T-

850 - Tenth Rib Height - The vertical distance between a standing surface and the tenth rib landmark at the bottom of the right side of the rib cage is measured with an anthropometer.  The subject stands erect looking straight ahead.  The shoulders and upper extremities are relaxed.

851 - Thigh Circumference (Distal) - The circumference of the right thigh just above the knee.

852 - Thigh Circumference (Proximal) - The circumference of the right thigh at its juncture with the buttock.  Measured with a tape.  The measurement is made perpendicular to the long axis of the thigh.  The subject stands erect with the weight distributed equally on both feet.  The legs are spread apart just enough so that the thighs do not touch.

853 - Thigh Circumference (Proximal - Sitting) - The circumference of the thigh as measured as high in the crotch as possible.  The measurement is made in a plane perpendicular to the long axis of the upper leg.

856 - Thigh Clearance - The vertical distance between a sitting surface and the highest point on the top of the right thigh.  Measured with an anthropometer.  The subject sits with the thighs parallel, knees flexed 90 and the feet in line with the thighs.

859 - Thigh-Thigh Breadth (Sitting) - The maximum breadth across the thighs.

862 - Thumb Breadth - The maximum breadth of the right thumb perpendicular to its long axis is measured with a Holtain caliper.  The thumb is held straight and held away from the hand at about a 45 angle.

863 - Thumb-Crotch Length - The distance from the skinfold at the base of the thumb to the notch between the 1st and 2nd fingers, measured with calipers held parallel to the main axis of the head.

867 - Thumb-Tip Reach - The horizontal distance from a back wall to the tip of the right thumb.  Measured on a wall scale.  Subject stands erect in a corner looking straight ahead with the feet together and the heels 20 cm from the back wall.  The buttocks and shoulders are against the wall.  The right arm and hand, palm down, are stretched forward horizontally along a scale on the side wall.  The thumb continues the horizontal line of the arm and the index finger curves around to touch the pad at the end of the thumb.  The subject’s right shoulder is held against the rear wall.

868 - Thumb-Tip Reach (Sitting) - Subject is seated in an erect position with his or her back against a wall.  The arm is extended parallel to the floor with the thumb parallel to the arm and the forefinger in opposition.  The distance between the arm and the tip of the thumb and forefinger is recorded.

869 - Thumb-Tip Reach  (Extended) - The distance from the wall to the tip of the right thumb measured with the left shoulder extended as far as possible, the arm held horizontal and the tip of the index finger touching the pad of the thumb.

873 - Tibiale Height - The height of the proximal medial margin of the tibia.

877 - Tragion Height. - The height of the tragion (cartilage flap on the ear).

880 - Tragion to Occiput - The distance from the tragion to the back of the head.

882 - Tragion to Top of Head (Vertex) - The distance from the tragion to the level of the top of the head.

884 - Tragion to Wall - The distance from the tragion to the wall.

888 - Triceps Skinfold - The thickness of a skinfold on the back of the arm halfway between acromion and the tip of the elbow, picked up parallel to the long axis of the upper arm.

890 - Triceps Skinfold II - The previous measurement repeated on the opposite arm.

894 - Trochanterion Height - The vertical distance between a standing surface and the trochanterion landmark on the upper side of the right thigh.  Measured with an anthropometer.  The subject stands erect looking straight ahead.  The heels are together with the weight distributed equally on both feet.

896 - Trunk Depth - The depth of the chest below the bust or the abdomen, whichever is larger.

898 - Trunk Height - The height of the suprasternale above the sitting surface.

-V-

914 - Vertical Trunk Circumference-Midbuttock (ASCC) - The vertical circumference of the torso is measured with a tape passing between the buttocks, to the right of the genitalia, over the right bustpoint landmark on women or the nipple (thelion) on men, and across the midshoulder landmark.  The subject stands erect looking straight ahead with the right arm hanging relaxed at the side.  The measurement is taken at the midpoint of quiet respiration.

916 - Vertical Trunk Circumference (USA) - The vertical circumference of the torso is measured with a tape passing over the maximum protrusion of the right buttock, to the right of the genitalia, over the right bustpoint landmark on women or the nipple (thelion) on men, and across the midshoulder landmark.  The subject stands erect looking straight ahead with the right arm hanging relaxed at the side.  The measurement is taken at the maximum point of quiet respiration.

917 - Vertical Trunk Circumference (sitting) - The previous measurement repeated with the subject sitting

-W-

919 - Waist Back Length (Natural) - The surface distance between the cervicale landmark on the back of the neck and the posterior-waist (natural indentation) landmark.  Measured with a tape.  The subject stands erect with the head in the Frankfort plane.  The shoulders and upper extremities are relaxed.

921 - Waist Back Length (Omphalion) - The surface distance between the cervicale landmark at the back of the neck and the posterior-waist (omphalion) landmark at the level of the navel.  Measured with a tape.  The subject stands erect with the head in the Frankfort plane.  The shoulders and upper extremities are relaxed.

924 - Waist Breadth (Natural) - Breadth of the waist at the level of the natural indentation of the waist.

928 - Waist Breadth (Omphalion) - The horizontal breadth of the waist at the level of the omphalion.  Measured with a beam caliper.  The subject stands erect looking straight ahead.

931 - Waist Circumference (Natural) - The horizontal circumference of the waist at the level of its natural indentation.  measured with a tape passing over right and left waist (natural indentation) landmarks.  The subject stands erect looking straight ahead.

932 - Waist Circumference (Omphalion) - The horizontal circumference of the waist at the level of the center of the navel (omphalion).  Measured with a tape.  The subject stands erect looking straight ahead.  The heels are together with the weight distributed equally on both feet.

935 - Waist Circumference (Omphalion-sitting) - The previous measure repeated with the subject sitting.

939 - Waist Depth (Natural) - The depth of the waist at the level of the natural indentation.

943 - Waist Depth (Omphalion) - The horizontal distance between the front and back of the waist at the level of the center of the navel (omphalion).  Measured with a beam caliper.  The subject stands erect looking straight ahead.  The heels are together with the weight distributed equally on both feet.

945 - Waist Front Length (Natural) -  The surface distance between the anterior-neck landmark and the anterior-waist (natural indentation) landmark.  Measured with a tape.  The subject stands erect with the head in a Frankfort plane.  The measurement is made at the maximum point of quiet respiration.

946 - Waist Front Length (Omphalion) - The surface distance between the anterior-neck landmark and the center of the navel (omphalion).  Measured with a tape.  The subject stands erect with the head in the Frankfort plane.

947 - Waist Hip Length - The surface distance between the right waist (omphalion) landmark and the right lateral-buttock point landmark on the side of the hip.  Measured with a tape.  The subject stands erect looking straight ahead.

949 - Waist Height (Natural) - The vertical distance between a standing surface and the landmark at the natural indentation of the right waist.  Measured with an anthropometer.  The subject stands erect looking straight ahead.  The heels are together with the weight distributed equally on both feet.  The shoulders and upper extremities are relaxed.

950 - Waist Height (Omphalion) - The vertical distance between a standing surface and the center of the navel (omphalion).  Measured with an anthropometer.  The subject stands erect looking straight ahead.  The heels are together with the weight distributed equally on both feet.  The shoulders and upper extremities are relaxed.

951 - Waist Height (Natural - Sitting) - The vertical distance from a sitting surface to the landmark at the natural indentation of the right waist.  Measured with an anthropometer.  The subject sits erect looking straight ahead.  The knees are flexed 90.

952 - Waist Height Sitting (Umbilicus) -The vertical distance from a sitting surface to the center of the navel (omphalion).  Measured with an anthropometer.  The subject sits erect looking straight ahead.  The knees are flexed 90.

954 - Waist Natural to Waist - Umbilicus - The surface distance between the right waist (natural indentation) and the right waist (omphalion).  Measured with a tape.  Subject stands erect looking straight ahead.  The heels are together with the weight distributed equally on both feet.

957 - Weight - The weight of the subject taken to the nearest tenth of a kilogram.  Subject stands on the platform of a scale.

958 - Weight (Clothed) - The weight of the subject wearing normal indoor apparel.

960 - Weight (Reported) - Weight as reported by the subject immediately prior to being measured.

964 - Wrist Breadth (Bone) - The distance between the radial and ulnar styloid prominence of the wrist measured with the flesh compressed.

967 - Wrist Circumference (Minimum) - The measurement taken as the minimum circumference between the styloid processes of the radius and the ulna and the carpals.  Left side only.

970 - Wrist Circumference (Stylion) - The circumference of the wrist perpendicular to the long axis of the forearm.  Measured with a tape passing over the stylion landmark on the wrist.  The subject extends the right arm forward with the palm up.

973 - Wrist Height - The vertical distance between a standing surface and the stylion landmark on the right wrist.  Measured with an anthropometer.  Subject stands erect looking straight ahead.  Shoulders are relaxed and the arms are extended downwards with the elbow, wrist, and fingers held rigidly straight.  The arms lightly touch the sides.

974 - Wrist Height (Sitting) - The vertical distance between the floor and the stylion landmark on the right wrist of a seated subject.  Measured with an anthropometer.  Subject sits erect looking straight ahead.  The shoulders are relaxed and the arms are extended downwards with the elbow, wrist, and fingers held rigidly straight.  The arms lightly touch the sides.

976 - Wrist to Center of Grip Length - The horizontal distance between the stylion landmark on the right wrist and the center of a dowel (1 1/4” diameter) gripped in the right hand.  Measured with a Poech caliper.  The subject sits grasping a dowel in the right hand.  The base of the dowel is flush with the bottom of the fist.  The subject puts the bottom of the fist on a flat surface in such a way that the base of the dowel rests on the surface.  The fist is in line with the long axis of the forearm.

977 - Wrist to Index Finger Length - The distance between the stylion landmark on the right wrist and the tip of the right index finger.  Measured with a Poech caliper.  The subject places the palm on a table, the fingers together, and the thumb abducted.  The middle finger is parallel to the long axis of the forearm.

978 - Wrist to Thumbtip Length - The horizontal distance between the stylion landmark on the right wrist and the tip of the right thumb is measured with a Poech caliper.  The subject rests the little finger side of the hand on a flat surface.  The thumb is held straight and in line with the long axis of the forearm.  The thumb rests on the first knuckle of the curved index finger.

979 - Wrist to Wall Length - The horizontal distance between a back wall and the stylion landmark on the right wrist of the outstretched arm.  Measured on a wall scale.  Subject stands erect in a corner looking straight ahead with the feet together and the heels 20 cm from the back wall.  The buttocks and shoulders are against the wall.  The right arm and hand with the palm down are stretched forward horizontally against a scale on the side wall.  The thumb continues the horizontal line of the arm and the index finger curves around to touch the pad at the end of the thumb.  The subject’s right shoulder is held against the rear wall.

980 - Wrist to Wall Length (Extended) - The horizontal distance between a back wall and the stylion landmark on the right wrist of the maximally outstretched arm.  Measured on a wall scale.  Subject stands erect in a corner looking straight ahead with the feet together and the heels 20 cm from the back wall.  The buttocks and shoulders are against the wall.  The right arm and hand with the palm down are stretched forward horizontally as far as possible against the side wall.  The thumb continues the horizontal line of the arm and the index finger curves around to touch the pad at the end of the thumb.  The subject’s left shoulder is held against the rear wall.

STRENGTH MEASUREMENTS

Nine sets of static strength measurements were made.  Three types of handles were used for these measures: a long handle, a short handle, and a D-ring.  The grips of both handles and the D-ring were metal cylinders approximately 2.5 centimeters in diameter around which several layers of tape were wound.  All measures were made at fixed distances above the ground.

Forces were measured in pounds on a strain gauge and recorded and displayed on a lifting force meter which recorded a peak and average strength score.  For each type of measurement, four measures were recorded:  Average 1, Average 2, Peak 1, and Peak 2.

1501 - 1504 -Strength - One hand Pull, Seated - 45 cm.  Subject sits erect, with feet 55 cm apart.  Using the dominant hand, the subject is instructed to grasp, from the underside, a D-ring that is attached 45 cm above the platform just forward of the chair in the vertical plane.  Keeping the shoulders square with the non-dominant arm placed on the lap and feet firmly planted on the platform, the subject is to pull the D-ring upward.

1505 - 1508 - Strength - Side, One hand Pull, Seated - 45 cm.  Subject sits erect, with feet 55 cm apart.  Using the dominant hand, the subject is instructed to grasp, from the underside, a D-ring that is attached 45 cm above the platform and a short distance to the right (or left, as is appropriate).  Keeping the shoulders square with the non-dominant arm placed on the lap and feet firmly planted on the platform, the subject is to pull the D-ring upward

1509 - 1512 - Strength, One hand Pull, Standing, 100 cm.  Subject stands erect, with feet 15 cm apart.  Using the dominant hand (right, if ambidextrous), the subject is instructed to grasp, from the underside, a D-ring that is attached 100 cm above the platform and at a point just to the right (or left, as is appropriate) of his or her body.  Keeping the shoulders square with the non-dominant arm relaxed and the feet firmly planted on the platform, the subject is to pull the D-ring upward.

1513 - 1516 - Strength, Two hand Pull, Seated - 38 cm.  Subject sits erect, with feet 55 cm apart.  Using both hands, the subject is instructed to grasp a short handle that is attached 38 cm above the platform at a point just forward of the chair in the vertical plane.  Leaning slightly forward and primarily using the arms and shoulders while keeping the feet flat, the subject is instructed to pull upward on the handle.

1517 - 1520 - Strength, Two hand Pull, Seated - 50 cm.  Subject sits erect, with feet 55 cm apart.  Using both hands, the subject is instructed to grasp a short handle that is attached 50 cm above the platform at a point just forward of the chair in the vertical plane.  Leaning slightly forward and primarily using the arms and shoulders while keeping the feet flat, the subject is instructed to pull upward on the handle.

1521 - 1524 - Strength - Two hand Pull, Standing - 38 cm.  Subject stands with feet 45 cm apart.  Using both hands, the subject is instructed to grasp a short handle that is attached 38 cm above the platform and directly in front of the subject.  Bending at the waist and primarily using the arms and shoulders, but also using the legs by extending them upward, the subject is instructed to pull upward on the handle.

1525 - 1528 - Strength - Two hand Pull, Standing - 50 cm.  Subject stands with feet 45 cm apart and knees straight.  Using both hands, the subject is instructed to grasp a long handle that is attached 50 cm above the platform and directly in front of the subject.  Bending at the waist and primarily using the arms and shoulder, the subject is instructed to pull upward on the handle.

1529 - 1532 - Strength - Two hand Pull, Standing - 100 cm.  Subject stands erect, with feet 45 cm apart.  Using both hands, the subject is instructed to grasp a long handle that is attached 100 cm above the platform and directly in front of the subject.  While keeping the knees straight and the feet firmly planted on the platform, the subject is instructed to pull upward on the handle.

1533 - 1536  - Strength - Two hand Push, Standing - 150 cm.  Subject stands erect, with feet 45 cm apart.  Using both hands, the subject is instructed to grasp a long handle that is attached 150 cm above the platform and directly in front of the subject.  While keeping the knees straight and the feet firmly planted on the platform, the subject is instructed to push the handle straight upward using the arms and shoulders.

###### LANDMARK GOLSSARY

-A-

- Abdominal - Pertaining to the abdomen, particularly the region below the rib cage and above the pelvis.

- Acromial - Pertaining to acromion, a point on the shoulder, which is also called acromiale.

- Acromiale - The highest point on the lateral edge of the acromial process of the scapula; also called the acromion.

- Ankle Height - The height of the marked point indicating the smallest horizontal circumference of the ankle.

- Anterior - Pertaining to the front of the body.

- Anterior Neck Point - The point in the midsagittal plane where the ascending column of the neck intersects the horizontal flesh under the jaw.  This point may be obscured by fat.

- Auricular - Pertaining to the ear.

- Axillary - Pertaining to the region of the axilla or armpit.

-B-

- Base of Hand - The line of demarcation drawn between the hand and the lower arm.  This line usually coincides with the skin crease at the proximal edge of the palm.

- Base of Thumb - The point at which the extended base of hand line intersects the radial border of the wrist.

- Biceps - The large muscle on the anterior side of the upper arm.

- Brow Ridges - The bony elevations, just above the orbits, on the frontal bone of the skull.

- Buttock Height - The height of the landmark that indicates the maximum posterior protrusion of the right buttock.

-C-

- Calf Height - The height of the landmark that indicates the maximum circumference of the right calf.

- Canthus - The corner or angle formed by the meeting of the eyelids.

- Cervicale - The protrusion of the spinal column at the base of the neck, caused by the dorsal tip of the spine of the 7th cervical vertebrae.

- Cornea - The transparent outer layer of the eyeball in the anterior plane.

- Coronal Plane- A vertical, lateral plane parallel to the coronal suture of the skull.

- Crinion - The point in the midplane where the hairline meets the forehead.

-D-

- Dactylion - The point, when the subject stands erect with extended and hand by his side, at the distal tip of the middle finger.

- Deltoid - The widest protrusion of the deltoid muscle.

- Deltoid Muscle - The large muscle on the lateral border of the upper arm in the shoulder region.

- Distal - The end of a body segment farthest from the head; opposed to proximal.

-E-

- Epicondyle - Primarily the lateral, but also the medial, protrusion of the humerus at the elbow.

- External - Away from the midsagittal plane of the body; lateral; opposed to medial.

- External Canthus - The external corner or angle formed by the juncture of the upper and lower eye lid.

-F-

- Frankfort Plane - The standard horizontal plane of orientation of the head, containing tragion and the lowest point of the orbit.  This is closely approximated when the subject looks directly forward with his or her line of vision parallel with the floor.

-G-

- Glabella - The most anterior point of the forehead between the brow ridges, in the midsagittal plane.

- Gluteal Furrow - The furrow formed by the protrusion of the buttock beyond the back of the leg.

- Gonial Angle - The obtuse angle at the back of the lower jaw formed by the intersection of the vertical and horizontal portions of the jaw.

-H-

- Helix - The rolled outer portion of the ear.

- Humerus - The bone of the upper arm.

-I-

- Inion - A small bony bump often found at the rearmost part of the head.

- Inseam - A tailoring term indicating the inside length of sleeve or trouser, measured on the medial side of the arm or leg.

- Instep Throat - The anterior juncture of the ankle with the right foot as determined by palpation and visual inspection.

- Internal - Near the midsagittal plane of the body.

-K-

- Knuckle - The joint formed by the meeting of a finger bone (phalanx) with a bone of the palm of the hand (metacarpal).

-L-

- Larynx - The cartilaginous walled cavity at the top of the windpipe in the front of the neck which contains the vocal cords.

- Lateral - Lying toward the sides of the body; opposed to medial.

- Lower Arm Circumference Level - The level of maximum protrusion of the brachio-radialis muscle when the arm is flexed, and the subject is making a fist.

- Lower Thigh Level - The level above the knee of the lower edge of the vastus lateralis muscle when the thigh is relaxed.

-M-

- Malleolar - Referring to the malleolus.

- Malleolus - The bony protrusion, either lateral or medial, of the ankle.

- Mandible - The lower jaw.

- Mastoid Process - The bony protrusion directly behind the ear.

- Medial - Lying near the midsagittal plane of the body; opposed to lateral.

- Menton - The lowest point of the tip of the chin in the midsagittal plane.

- Metacarpal Bone - A bone of the palm of the hand.

- Metacarpale - The joint of juncture of a bone of the palm (metacarpal) with the first bone (phalanx) of the finger.

- Metatarsal - A bone of the instep of the foot.

- Metatarsal-Phalageal Joint - The distal junction of the 1st metatarsal with its phalanx as determined by palpation.

- Mid-Shoulder - A point on the superior border of the shoulder midway between acromion and the juncture of the neck and shoulder.

- Midaxillary - A vertical line originating from the center of the axillary (armpit) space as determined by visual inspection.

- Midplane - Same as midsagittal plane.

- Midsagittal Plane - The vertical plane which divides the body into symmetrical right and left sections.

- Minimum Frontal Points - The points on the left and right sides of the forehead which immediately overlie the deepest indentation of the temporal crests, just above the lateral ends of the brow ridges.

-N-

- Nasal Root Depression - The area of greatest indentation where the bridge of the nose meets the forehead.  (also called the sellion)

- Nasal Septum - The cartilaginous wall separating the right nostril from the left.

- Natural Waist Line - The level of greatest lateral indentation to the abdomen region.  If no Natural Waist Line is visible the level at which the belt is worn is used instead.

- Navicular Bone - The small bone of the hand just distal to the bend of the wrist or the base of the thumb.

- Neck Point - The point marking the intersection of the right side of the neck with the trapezius muscle of the right shoulder.  This may be difficult to locate accurately in either corpulent or underdeveloped individuals.

- Nuchale - The lowest point in the midsagittal plane of the occiput that can be palpated among the nuchal muscles.  This point is often visually obscured by hair.

-O-

- Occipital Region - The back of the head.

- Occiput - The bone at the back of the skull; the region of the back of the head.

- Ocular - Of or pertaining to the eyes.

- Olecranon - The bony tip of the elbow.

- Omphalion - The midpoint of the umbilicus or navel.

-P-

- Patella - The kneecap.

- Patella (top) - The superior edge of the knee cap as determined by palpation.

- Phalangeal - Pertaining to a phalanx or the phalanges, referring to the fingers or toes.

- Phalanx - (plural, phalanges) a bone of the fingers or toes.

- Philtrum - The vertical groove running from the upper membranous lip to the base of the nasal septum.

- Popliteal Area - The hollowed out region of the leg directly behind the knee, involving both the bottom of the thigh and the top of the calf.

- Posterior - Pertaining to the back of the body.

- Pronasale - The most anterior tip of the nose.

- Proximal - The end of a body segment nearest the head; opposed to distal.

- Pupillary - Pertaining to the pupil of the eye.

-R-

- Radiale - The highest point on the proximal head of the radius, near the midpoint of the elbow joint on the posterior side of the arm.

- Radial - Pertaining to the radius.

- Radius - One of the two bones of the lower arm.  This bone extends from the lateral side of the elbow to the wrist at the base of the thumb.

- Ramus - The vertical portion of the lower jaw bone (mandibular).

-S-

- Scye - The point marking the upper end of the posterior axillary fold, either right or left, which is the furrow formed by the juncture of the upper arm and upper back.

- Sellion - The point of maximum indentation where the bridge of the nose meets the forehead (also called the Nasal Root Depression).

- Sits Erect - Subject sits on a flat horizontal surface, his or her weight distributed equally, with his or her back held in and his shoulders held back.  This position requires holding the torso straight but not rigid.

- Sphyrion - The most distal point on the tibia;  it lies at the tip of the malleolar process.

- Stands Erect - Subject stands on a flat surface, his or her weight distributed equally, with the back held in and the shoulders held back and the legs fully straightened.  This position requires holding the body straight but not rigid.

- Sternum - The breastbone.

- Stomion - The point of contact in the midsagittal plane between the upper and lower lips.

- Stylion - The point at the anterior distal end of the radius.

- Submandibular - Pertaining to the region under the mandible or lower jaw.

- Subnasale - The point where the base of the nasal septum meets the philtrum.

- Suprasternale - The lowest point of the jugular notch on the sternum as determined by palpation.

-T-

- Temporal Region - The area on the side of the head between and above the eyes and ears.

- Temporal Crest - A narrow, bony ridge running along the side of the head, curving up from the upper lateral margin of the eye socket, above and past the ear and downward, ending behind the ear.  This serves as the area of attachment for the temporal muscles.

- Temporal Muscles - The muscles of the temple region.

- Tibiale - The uppermost point on the medial superior surface of the tibia.

- Thelion - The right nipple.

- Tragion - The point located at the notch just above the tragus of the ear.  This point corresponds approximately to the upper edge of the ear hole.

- Tragus - The small cartilaginous flap of flesh in front of the ear hole.

- Trapezius Muscle - The large muscle at the back of the neck and shoulder.

- Trochanterion - The top of the bony lateral protrusion of the proximal end of the femur.

-U-

- Ulna - One of the two bones of the lower arm;  this bone runs from the tip of the elbow to the wrist on the same side as the little finger

- Ulnar - Referring to the ulna.

-V-

- Vertex - The top of the head in the midsagittal plane when the head is held in the Frankfort Plane.

-W-

- Waist Height (Omphalion) - The height of the horizontal plane through the omphalion, or the middle of the umbilicus.

- Wrist - The styloid process of the ulna (center of the small bone which protrudes at the outside of the wrist).

-Z-

- Zygomatic Arch - The bony arch extending horizontally along the side of the head from the cheekbone nearly to the external ear.

##### LIST OF MEASUREMENTS

| CODE | MEASURE |
| --- | --- |
| 6 | ABDOMINAL DEPTH-SITTING |
| 7 | ABDOMINAL-EXT-BRTH-SITTING |
| 8 | ABDOMINAL-EXT-CIRC |
| 10 | ABDOMINAL-EXT-DEPTH |
| 11 | ABDOMINAL-EXT-DEPTH-SIT |
| 18 | ABDOMINAL-EXT-HT |
| 23 | ACROMION HT |
| 25 | ACROMION HT - SITTING |
| 39 | ACROMION-RADIALE LNTH |
| 42 | ACROMION TO BICEPS CIRC LEVEL |
| 48 | AGE |
| 58 | ANKLE CIRC |
| 64 | ANKLE HT |
| 70 | ARM CIRC-AXILLARY |
| 72 | ARM CIRC-BICEPS RELAXED |
| 73 | ARM CIRC-BICEPS RELAXED-LEFT |
| 77 | ARM CIRC-BICEPS FLEXED |
| 78 | ARM CIRC-BICEPS FLEXED-LEFT |
| 80 | ARM REACH FROM WALL |
| 81 | ARM REACH FROM WALL - SITTING |
| 90 | AXILLA HT |
| 91 | AXILLA TO WAIST |
| 92 | BACK CURVATURE |
| 93 | BACK ARC - BUST |
| 95 | BACK ARC- WAIST |
| 103 | BIACROMIAL BRTH |
| 110 | BICEPS SKF |
| 122 | BIDELTOID BRTH |
| 126 | BIGONIAL BRTH |
| 130 | BIILIOCRISTALE BRTH |
| 134 | BIMALLEOLAR BRTH |
| 138 | BIOCULAR BRTH |
| 140 | BISPINOUS BRTH |
| 142 | BITRAGION BRTH |
| 144 | BITRAGION-CORONAL ARC |
| 146 | BITRAGION-CRINION ARC |
| 150 | BITRAGION-MENTON ARC |
| 152 | BITRAGION-MINIMUM FRONTAL ARC |
| 153 | BITRAGION-POSTERIOR ARC |
| 156 | BITRAGION-SUBMANDIBULAR ARC |
| 158 | BITRAGION-SUBNASALE ARC |
| 161 | BITROCHANTERION BRTH |
| 165 | BIZYGOMATIC BRTH |
| 167 | BODY LNTH-SUPINE-KNEES FLEXED |
| 169 | BOWED TORSO HT |
| 172 | BUSTPOINT TO BUSTPOINT BRTH |
| 178 | BUTTOCK CIRC |
| 179 | BUTTOCK CIRC-SITTING |
| 185 | BUTTOCK CURVATURE |
| 186 | BUTTOCK DEPTH |
| 187 | BUTTOCK-HEEL LNTH |
| 188 | BUTTOCK HT |
| 194 | BUTTOCK-KNEE LNTH |
| 200 | BUTTOCK-POPLITEAL LNTH |
| 207 | CALF CIRC |
| 209 | CALF CIRC-LEFT |
| 215 | CALF HT |
| 219 | CERVICALE HT |
| 220 | CERVICALE HT-SITTING |
| 223 | CHEST BRTH |
| 227 | CHEST BRTH-BONE |
| 230 | CHEST CIRC |
| 231 | CHEST CIRC-AT SCYE |
| 232 | CHEST CIRC-BELOW BUST |
| 233 | CHEST CIRC-INSPIRATION |
| 235 | CHEST DEPTH-SITTING |
| 236 | CHEST DEPTH |
| 237 | CHEST HT |
| 249 | CROTCH HT |
| 251 | CROTCH LNTH (NATURAL WAIST) |
| 252 | CROTCH LNTH-UMBILICUS |
| 253 | CROTCH LNTH-POSTERIOR-NATURAL |
| 254 | CROTCH LNTH-POSTERIOR-OMPHALION |
| 265 | DACTYLION HT |
| 269 | DELTOID ARC |
| 277 | EAR BRTH |
| 280 | EAR LNTH |
| 282 | EAR LNTH-ABOVE TRAGION |
| 285 | EAR PROTRUSION |
| 286 | EAR TO EAR BRTH |
| 293 | ELBOW BRTH-BONE |
| 297 | ELBOW BRTH-BONE LEFT |
| 301 | ELBOW CIRC-FULLY BENT |
| 303 | ELBOW CIRC-EXTENDED |
| 305 | ELBOW CIRC-FLEXED |
| 307 | ELBOW-GRIP LNTH |
| 309 | ELBOW HT |
| 312 | ELBOW REST HT |
| 318 | ELBOW-ELBOW BRTH |
| 321 | ELBOW FUNCTIONAL REACH |
| 324 | ELBOW-WRIST LNTH |
| 325 | ELBOW SPAN |
| 326 | EXTERNAL CANTHUS TO OCCIPUT |
| 327 | EXTERNAL CANTHUS TO VERTEX |
| 328 | EXTERNAL CANTHUS TO WALL |
| 329 | EYE HEIGHT-STANDING |
| 330 | EYE HT-SITTING |
| 340 | FIBULA HT |
| 344 | FIBULA HT |
| 348 | FIRST PHALANX III LENGTH |
| 356 | FOOT BRTH |
| 358 | FOOT CIRC |
| 362 | FOOT LNTH |
| 370 | FOREARM CIRC-EXTENDED |
| 371 | FOREARM CIRC-FLEXED |
| 375 | FOREARM-HAND LNTH |
| 378 | FOREARM TO FOREARM BRTH |
| 386 | FUNCTIONAL LEG LNTH |
| 391 | GLABELLA TO OCCIPUT |
| 393 | GLABELLA TO TOP OF HEAD |
| 394 | GLABELLA TO WALL |
| 395 | GLUTEAL ARC |
| 402 | GRIP DIAMETER (INSIDE) |
| 404 | GRIP DIAMETER (OUTSIDE) |
| 398 | GLUTEAL-FURROW HT |
| 402 | GRIP DIAMETER (INSIDE) |
| 404 | GRIP DIAMETER (OUTSIDE) |
| 405 | GRIP REACH-OVERHEAD |
| 407 | GRIP STRENGTH |
| 411 | HAND BRTH AT METACARPALE |
| 413 | HAND BRTH-AT THUMB |
| 416 | HAND CIRC AT METACARPALE |
| 417 | HAND CIRC-OVER THUMB |
| 420 | HAND LNTH |
| 424 | HAND SKF |
| 425 | HAND THICKNESS AT METACARPALE III |
| 427 | HEAD BRTH |
| 430 | HEAD CIRC |
| 433 | HEAD DIAG-INION TO PRONASALE |
| 435 | HEAD DIAG-MAX FROM MENTON |
| 436 | HEAD DIAG-MAX FROM NUCHALE |
| 437 | HEAD DIAG-MENTON TO OCCIPUT |
| 441 | HEAD LNTH |
| 445 | HEEL ANKLE CIRC |
| 450 | HEEL BRTH |
| 457 | HIP BRTH |
| 459 | HIP BRTH-SITTING |
| 465 | HIP CIRC AT TROCHANTERION |
| 466 | HIP CIRC-7 BELOW WAIST |
| 468 | HIP CIRC-9 BELOW WAIST |
| 489 | ILIOCRISTALE HT |
| 490 | ILIOSPINALE HT |
| 493 | INSTEP CIRC |
| 496 | INSTEP LNTH |
| 500 | INTEROCULAR DIST |
| 503 | INTERPUPILLARY DIST |
| 506 | INTERSCYE DIST |
| 507 | INTERSCYE DIST-MAX |
| 508 | INTERSCYE DIST-MID |
| 510 | INTERSCY - FRONT |
| 511 | JUXTANIPPLE SKF |
| 512 | KNEE BRTH-BONE |
| 513 | KNEE BRTH BONE-LEFT |
| 515 | KNEE CIRC |
| 516 | KNEE CIRC-FULLY BENT |
| 517 | KNEE CIRC - SITTING |
| 528 | KNEE HT-FLEXED SUPINE |
| 529 | KNEE HT - SITTING |
| 536 | KNEE-KNEE BRTH |
| 538 | KNEELING HT |
| 540 | KNEELING-LEG LNTH |
| 542 | LATERAL FEMORAL EPICONDYLE HT |
| 543 | LATERAL-MALLEOUS HT |
| 547 | LIP LNTH |
| 549 | LIP LNTH-SMILING |
| 552 | LIP PROTRUSION TO OCCIPUT |
| 553 | LIP PROTRUSION TO WALL |
| 555 | LIP TO LIP LNTH |
| 569 | MAX FRONTAL BRTH |
| 572 | MAX REACH FROM WALL |
| 576 | MEDIAL CALF SKF |
| 579 | MEDIAL MALLEOUS HT |
| 583 | MENTON-CRINION LNTH |
| 585 | MENTON PROJECTION |
| 586 | MENTON TO NASAL ROOT DEP LNTH |
| 588 | MENTON TO OCCIPUT |
| 592 | MENTON-SUBNASALE LNTH |
| 595 | MENTON TO TOP OF HEAD |
| 596 | MENTON TO WALL |
| 601 | METACARPALE HT |
| 612 | MIDSHOULDER HT-SITTING |
| 615 | MID-THIGH CIRC |
| 616 | MINIMUM FRONTAL ARC |
| 618 | MIN FRONTAL BRTH |
| 625 | NASAL ROOT BRTH |
| 629 | NASAL ROOT HT |
| 631 | NASAL ROOT DEPRESSION TO OCCIPUT |
| 633 | NASAL ROOT DEPRESSION TO TOP OF HEAD |
| 634 | NASAL ROOT DEPRESSION TO WALL |
| 636 | NECK CIRC-BASE |
| 637 | NECK CIRC-BELOW LARYNX |
| 639 | NECK CIRC-OVER LARYNX |
| 641 | NECK HT-LATERAL |
| 644 | NECK LNTH-ANTERIOR |
| 647 | NECK LNTH-POST-NUCHALE |
| 648 | NECK TO BUSTPOINT LNTH |
| 649 | NOSE LNTH |
| 651 | NOSE BRTH |
| 652 | NOSE PROTRUSION |
| 653 | OVERHEAD REACH |
| 654 | OVERHEAD REACH SITTING |
| 655 | OVERHEAD REACH-FORWARD |
| 656 | OVERHEAD REACH-EXTENDED |
| 657 | PALM LNTH |
| 663 | PATELLA-BOTTOM HT |
| 666 | PATELLA-TOP HT |
| 670 | PENALE HT |
| 674 | PHILTRUM LNTH |
| 678 | POPLITEAL HT-SITTING |
| 690 | PRONASALE TO OCCIPUT |
| 692 | PRONASALE TO TOP OF HEAD |
| 694 | PRONASALE TO WALL |
| 698 | RADIALE-STYLION LNTH |
| 705 | SAGITTAL ARC-NUCHALE |
| 732 | SCYE CIRC-OVER ACROMION |
| 737 | SCYE DEPTH |
| 743 | SHOULDER BRTH-BOWED |
| 745 | SHOULDER BRTH-REACH OVERHEAD |
| 747 | SHOULDER CIRC |
| 751 | SHOULDER-ELBOW LNTH |
| 754 | SHOULDER LNTH |
| 758 | SITTING HT |
| 760 | SITTING HT-RELAXED |
| 764 | SLEEVE-INSEAM LNTH |
| 797 | SLEEVE-OUTSEAM LNTH |
| 799 | SPAN |
| 801 | SPHYRION HT |
| 802 | SPINE TO ELBOW LNTH (SL) |
| 803 | SPINE TO SCYE LNTH (SL) |
| 804 | SPINE TO WRIST LNTH (SL) |
| 805 | STATURE |
| 806 | STATURE-CLOTHED |
| 807 | STATURE-MAXIMUM |
| 808 | STATURE-REPORTED |
| 814 | STOMION TO OCCIPUT |
| 815 | STOMION TO TOP OF HEAD |
| 816 | STOMION TO WALL |
| 821 | STRAP LNTH |
| 825 | SUBNASALE TO NASAL ROOT DEP |
| 829 | SUBNASALE TO OCCIPUT |
| 830 | SUBNASALE TO TOP OF HEAD |
| 831 | SUBNASALE TO WALL |
| 833 | SUBSCAPULAR SKF |
| 834 | SUBSCAPULAR SKF II |
| 837 | SUBSTERNALE HT |
| 841 | SUPRASTERNALE HT |
| 844 | SUPRAILIAC SKF |
| 845 | SUPRAILIAC SKF II |
| 848 | SUPRAPATELLA SKF |
| 850 | TENTH RIB |
| 851 | THIGH CIRC-DISTAL |
| 852 | THIGH CIRC-PROXIMAL |
| 853 | THIGH CIRC-PROXIMAL-SIT |
| 856 | THIGH CLEARANCE |
| 859 | THIGH TO THIGH BRTH-SIT |
| 862 | THUMB BRTH |
| 863 | THUMB-CROTCH LNTH |
| 867 | THUMB-TIP REACH |
| 868 | THUMB-TIP REACH-SITTING |
| 869 | THUMB-TIP REACH-EXTENDED |
| 873 | TIBIALE HT |
| 877 | TRAGION HT |
| 880 | TRAGION TO OCCIPUT |
| 882 | TRAGION TO TOP OF HEAD |
| 884 | TRAGION TO WALL |
| 888 | TRICEPS SKF |
| 890 | TRICEPS SKF II |
| 894 | TROCHANTERION HT |
| 896 | TRUNK DEPTH |
| 898 | TRUNK HT |
| 900 | UMBILICUS SKF |
| 914 | VERTICAL TRUNK CIRC-MIDBUTTOCK |
| 916 | VERTICAL TRUNK CIRC |
| 917 | VERTICAL TRUNK CIRC-SITTING |
| 919 | WAIST BACK LNTH-NATURAL |
| 921 | WAIST BACK LNTH-OMPHALION |
| 924 | WAIST BRTH-NATURAL |
| 928 | WAIST BRTH-OMPHALION |
| 931 | WAIST CIRC - NATURAL |
| 932 | WAIST CIRC-OMPHALION |
| 935 | WAIST CIRC-OMPHALION-SITTING |
| 939 | WAIST DEPTH-NATURAL |
| 943 | WAIST DEPTH-OMPHALION |
| 945 | WAIST FRONT LNTH-NATURAL |
| 946 | WAIST FRONT LNTH-OMPHALION |
| 947 | WAIST HIP LNTH |
| 949 | WAIST HT-NATURAL |
| 950 | WAIST HT-OMPHALION |
| 951 | WAIST HT-NATURAL-SITTING |
| 952 | WAIST HT-UMBILICUS-SITTING |
| 954 | WAIST NATURAL TO WAIST UMBILICUS |
| 957 | WEIGHT |
| 958 | WEIGHT-CLOTHED |
| 960 | WEIGHT-REPORTED |
| 964 | WRIST BRTH-BONE |
| 967 | WRIST CIRC-MIN |
| 970 | WRIST CIRC-STYLION |
| 973 | WRIST HT |
| 974 | WRIST HT-SITTING |
| 976 | WRIST TO CENTER OF GRIP LNTH |
| 977 | WRIST TO INDEX FINGER LNTH |
| 978 | WRIST TO THUMBTIP LNTH |
| 979 | WRIST TO WALL LNTH |
| 980 | WRIST TO WALL LNTH-EXTENDED |
| 1003 | BLOOD TYPE |
| 1004 | RH FACTOR |
| 1005 | HANDEDNESS |
| 1501 | STRENGTH-CENTER-SIT-ONE HAND PULL-45-AVG1 |
| 1502 | STRENGTH-CENTER-SIT-ONE HAND PULL-45-AVG2 |
| 1503 | STRENGTH-CENTER-SIT-ONE HAND PULL-45-PEAK1 |
| 1504 | STRENGTH-CENTER-SIT-ONE HAND PULL-45-PEAK2 |
| 1505 | STRENGTH-SIDE-SIT-ONE HAND PULL-45-AVG1 |
| 1506 | STRENGTH-SIDE-SIT-ONE HAND PULL-45-AVG2 |
| 1507 | STRENGTH-SIDE-SIT-ONE HAND PULL-45-PEAK1 |
| 1508 | STRENGTH-SIDE-SIT-ONE HAND PULL-45-PEAK2 |
| 1509 | STRENGTH-ONE HAND PULL-STD-100-AVG1 |
| 1510 | STRENGTH-ONE HAND PULL-STD-100-AVG2 |
| 1511 | STRENGTH-ONE HAND PULL-STD-100-PEAK1 |
| 1512 | STRENGTH-ONE HAND PULL-STD-100-PEAK2 |
| 1513 | STRENGTH-TWO HAND PULL-SIT-38-AVG1 |
| 1514 | STRENGTH-TWO HAND PULL-SIT-38-AVG2 |
| 1515 | STRENGTH-TWO HAND PULL-SIT-38-PEAK1 |
| 1516 | STRENGTH-TWO HAND PULL-SIT-38-PEAK2 |
| 1517 | STRENGTH-TWO HAND PULL- SIT-50-AVG1 |
| 1518 | STRENGTH-TWO HAND PULL-SIT-50-AVG2 |
| 1519 | STRENGTH-TWO HAND PULL-SIT-50-PEAK1 |
| 1520 | STRENGTH-TWO HAND PULL-SIT-50-PEAK2 |
| 1521 | STRENGTH-TWO HAND PULL-STD-38-AVG1 |
| 1522 | STRENGTH-TWO HAND PULL-STD-38-AVG2 |
| 1523 | STRENGTH-TWO HAND PULL-STD-38-PEAK1 |
| 1524 | STRENGTH-TWO HAND PULL-STD-38-PEAK2 |
| 1525 | STRENGTH-TWO HAND PULL-STD-50-AVG1 |
| 1526 | STRENGTH-TWO HAND PULL-STD-50-AVG2 |
| 1527 | STRENGTH-TWO HAND PULL-STD-50-PEAK1 |
| 1528 | STRENGTH-TWO HAND PULL-STD-50-PEAK2 |
| 1529 | STRENGTH-TWO HAND PULL-STD-100-AVG1 |
| 1530 | STRENGTH-TWO HAND PULL-STD-100-AVG2 |
| 1531 | STRENGTH-TWO HAND PULL-STD-100-PEAK1 |
| 1532 | STRENGTH-TWO HAND PULL-STD-100-PEAK2 |
| 1533 | STRENGTH-TWO HAND PUSH-STD-150-AVG1 |
| 1534 | STRENGTH-TWO HAND PUSH-STD-150-AVG2 |
| 1535 | STRENGTH-TWO HAND PUSH-STD-150-PEAK1 |
| 1536 | STRENGTH-TWO HAND PUSH-STD-150-PEAK2 |

SURVEY SPECIFIC MEASUREMENTS

| CODE | MEASURE |
| --- | --- |
| 1006 | BIRTHPLACE - SUBJECT - AIR FORCE FLYERS 1967 |
| 1007 | BIRTHPLACE - FATHER - AIR FORCE FLYERS 1967 |
| 1008 | BIRTHPLACE - MOTHER - AIR FORCE FLYERS 1967 |
| 1015 | AERORATING - AIR FORCE FLYERS 1967 |
| 1016 | AIRCRAFT CATEGORY - AIR FORCE FLYERS 1967 |
| 1017 | COMMAND - AIR FORCE FLYERS 1967 |
| 1023 | BOOT SIZE-LNTH - ARMY PERSONNEL 1966 |
| 1024 | BOOT SIZE-WDTH - ARMY PERSONNEL 1966 |
| 1025 | RANK - ARMY PERSONNEL 1966 |
| 1026 | PAY GRADE - ARMY PERSONNEL 1966 |
| 1027 | SERVICE - ARMY PERSONNEL 1966 |
| 1028 | EDUCATION - ARMY PERSONNEL 1966 |
| 1029 | MARITAL STATUS - ARMY PERSONNEL 1966 |
| 1030 | BIRTHPLACE - SUBJECT - ARMY PERSONNEL 1966 |
| 1031 | BIRTHPLACE - FATHER - ARMY PERSONNEL 1966 |
| 1032 | BIRTHPLACE - MOTHER - ARMY PERSONNEL 1966 |
| 1033 | RESIDENCE - LONGER - ARMY PERSONNEL 1966 |
| 1034 | NATIONAL EXTRACTION - ARMY PERSONNEL 1966 |
| 1035 | TOE BIGGEST - ARMY PERSONNEL 1966 |
| 1036 | GLASSES - ARMY PERSONNEL 1966 |
| 1037 | RACE - AIR FORCE FLYERS 1967 |
| 1038 | RANK - AIR FORCE FLYERS 1967 |
| 1041 | MOS - ARMY WOMEN 1977 |
| 1042 | RACE - ARMY WOMEN 1977 |
| 1043 | BIRTHDATE - ARMY WOMEN 1977 |
| 1044 | LENGTH OF SERVICE - ARMY WOMEN 1977 |
| 1045 | RANK - ARMY WOMEN 1977 |
| 1048 | RANK - NAVY FLYERS 1964 |
| 1049 | BIRTHPLACE - NAVY FLYERS 1964 |
| 1050 | AIRCRAFT - NAVY FLYERS 1964 |
| 1051 | LOCATION - NAVY FLYERS 1964 |
| 1056 | BIRTHPLACE - ARMY WOMEN 1977 |
| 1081 | RACE-HEALTH EXAM 1961 |
| 1082 | AGE AT MENARCH-HEALTH EXAM 1961 |
| 1210 | EDUCATION-HEALTH EXAM 1961 |
| 1083 | YRS OF SERVICE-CANADA 1974 |
| 1084 | RANK-CANADA 1974 |
| 1085 | UNIT-CANADA 1974 |
| 1086 | NATIONAL EXTRACTION-CANADA 1974 |
| 1087 | PRIMARY LANGUAGE-CANADA 1974 |
| 1090 | RANK-1970 ARMY AVIATORS |
| 1091 | AERORATING-1970 ARMY AVIATORS |
| 1092 | EDUCATION-1970 ARMY AVIATORS |
| 1093 | BIRTHPLACE-1970 ARMY AVIATORS |
| 1095 | GLASSES-1970 ARMY AVIATORS |
| 1096 | QUAL PARACHUTIST-1970 ARMY AVIATORS |
| 1097 | BIRTHDATE-1961 AIR TRAFFIC CONTROLLERS |
| 1098 | RACE-1961 AIR TRAFFIC CONTROLLERS |
| 1099 | AGE AT ENLISTMENT-1969 IMPERIAL IRANIAN ARMED FORCES |
| 1100 | GLASSESS-1969 IMPERIAL IRANIAN ARMED FORCES |
| 1101 | NORMAL VISION-1969 IMPERIAL IRANIAN ARMED FORCES |
| 1102 | COLOR PERCEPTION-1969 IMPERIAL IRANIAN ARMED FORCES |
| 1103 | RELATIVE TOE LENGTH-1969 IMPERIAL IRANIAN ARMED FORCES |
| 1104 | N OLDER BROS-1969 IMPERIAL IRANIAN ARMED FORCES |
| 1105 | N YOUNGER BROS.-1969 IMPERIAL IRANIAN ARMED FORCES |
| 1106 | N OLDER SIS.-1969 IMPERIAL IRANIAN ARMED FORCES |
| 1107 | N YOUNGER SIS.-1969 IMPERIAL IRANIAN ARMED FORCES |
| 1108 | N WIVES PRESENT-1969 IMPERIAL IRANIAN ARMED FORCES |
| 1109 | N WIVES DIVORCED-1969 IMPERIAL IRANIAN ARMED FORCES |
| 1110 | BIRTHPLACE-1969 IMPERIAL IRANIAN ARMED FORCES |
| 1111 | RES. AT ENLISTMENT-1969 IMPERIAL IRANIAN ARMED FORCES |
| 1112 | ETHNIC DERIVATION-1969 IMPERIAL IRANIAN ARMED FORCES |
| 1113 | 1ST LANGUAGE-1969 IMPERIAL IRANIAN ARMED FORCES |
| 1114 | 2ND LANGUAGE-1969 IMPERIAL IRANIAN ARMED FORCES |
| 1115 | 3RD LANGUAGE-1969 IMPERIAL IRANIAN ARMED FORCES |
| 1116 | MAKTAB-1969 IMPERIAL IRANIAN ARMED FORCES |
| 1117 | ELEMENTARY SCHOOL-1969 IMPERIAL IRANIAN ARMED FORCES |
| 1118 | HIGH SCHOOL-1969 IMPERIAL IRANIAN ARMED FORCES |
| 1119 | COLLEGE-1969 IMPERIAL IRANIAN ARMED FORCES |
| 1120 | BEYOND HS-1969 IMPERIAL IRANIAN ARMED FORCES |
| 1121 | OCCUPATION-1969 IMPERIAL IRANIAN ARMED FORCES |
| 1122 | SERVICE-1969 IMPERIAL IRANIAN ARMED FORCES |
| 1123 | LNTH OF SERVICE-1969 IMPERIAL IRANIAN ARMED FORCES |
| 1124 | RANK-1969 IMPERIAL IRANIAN ARMED FORCES |
| 1125 | MILITARY DUTY-1969 IMPERIAL IRANIAN ARMED FORCES |
| 1129 | LNTH OF SERVICE- LATIN AMERICAN MILITARY |
| 1130 | TOE LNTH- LATIN AMERICAN MILITARY |
| 1131 | LOCATION- LATIN AMERICAN MILITARY |
| 1132 | DATE MEASURED- LATIN AMERICAN MILITARY |
| 1133 | RANK- LATIN AMERICAN MILITARY |
| 1134 | BRANCH OF SERVICE- LATIN AMERICAN MILITARY |
| 1135 | EDUCATION- LATIN AMERICAN MILITARY |
| 1136 | N LANGUAGES SPOKEN- LATIN AMERICAN MILITARY |
| 1137 | 1ST LANGUAGE- LATIN AMERICAN MILITARY |
| 1138 | 2ND LANGUAGE- LATIN AMERICAN MILITARY |
| 1139 | 3RD LANGUAGE- LATIN AMERICAN MILITARY |
| 1140 | MARITAL STATUS- LATIN AMERICAN MILITARY |
| 1141 | TIME IN CANAL ZONE- LATIN AMERICAN MILITARY |
| 1142 | GLASSES-LATIN AMERICAN MILITARY |
| 1143 | REGION- LATIN AMERICAN MILITARY |
| 1144 | BOOT SIZE (LNTH)-1966 MARINE PERSONNEL |
| 1145 | BOOT SIZE (WDTH)-1966 MARINE PERSONNEL |
| 1146 | RANK-1966 MARINE PERSONNEL |
| 1147 | PAY GRADE-1966 MARINE PERSONNEL |
| 1148 | LNTH OF SERVICE-1966 MARINE PERSONNEL |
| 1149 | EDUCATION-1966 MARINE PERSONNEL |
| 1150 | MARITAL STATUS-1966 MARINE PERSONNEL |
| 1151 | BIRTHPLACE (SUBJ)-1966 MARINE PERSONNEL |
| 1152 | BIRTHPLACE (FATHER)-1966 MARINE PERSONNEL |
| 1153 | BIRTHPLACE (MOTHER)-1966 MARINE PERSONNEL |
| 1154 | LONGEST RESIDENCE-1966 MARINE PERSONNEL |
| 1155 | BIGGEST TOE-1966 MARINE PERSONNEL |
| 1156 | GLASSES-1966 MARINE PERSONNEL |
| 1157 | RANK- 1960-61 NATO SURVEY |
| 1158 | MILITARY ASSIGNMENT-1960-61 NATO SURVEY |
| 1159 | CURRENT DUTY-1971-1975 BRITISH ARMY |
| 1160 | CURRENT UNIT-1971-1975 BRITISH ARMY |
| 1161 | COUNTRY OF BIRTH-1971-1975 BRITISH ARMY |
| 1162 | COUNTY OF BIRTH-1971-1975 BRITISH ARMY |
| 1163 | BRANCH OF ARMY-1971-1975 BRITISH ARMY |
| 1164 | AIRCRAFT TYPE-1971 ROYAL AUSTRALIAN AIR FORCE |
| 1165 | AIRCREW CLASS-1971 ROYAL AUSTRALIAN AIR FORCE |
| 1166 | MARITAL STATUS-1972 AMERICAN AIRLINE STEWARDESSES |
| 1167 | N CHILDREN-1972 AMERICAN AIRLINE STEWARDESSES |
| 1168 | DAY OF BIRTH-1972 AMERICAN AIRLINE STEWARDESSES |
| 1169 | MONTH OF BIRTH-1972 AMERICAN AIRLINE STEWARDESSES |
| 1170 | YEAR OF BIRTH-1972 AMERICAN AIRLINE STEWARDESSES |
| 1171 | PLACE OF BIRTH-FATHER-1972 AMERICAN AIRLINE STEWARDESSES |
| 1172 | PLACE OF BIRTH-MOTHER-1972 AMERICAN AIRLINE STEWARDESSES |
| 1173 | PLACE OF BIRTH-SUBJECT-1972 AMERICAN AIRLINE STEWARDESSES |
| 1174 | RESIDENCY-1972 AMERICAN AIRLINE STEWARDESSES |
| 1175 | OLDER BROTHERS-1972 AMERICAN AIRLINE STEWARDESSES |
| 1176 | YOUNGER BROTHERS-1972 AMERICAN AIRLINE STEWARDESSES |
| 1177 | OLDER SISTERS-1972 AMERICAN AIRLINE STEWARDESSES |
| 1178 | YOUNGER SISTERS-1972 AMERICAN AIRLINE STEWARDESSES |
| 1179 | FOREIGN LANGUAGE 1-1972 AMERICAN AIRLINE STEWARDESSES |
| 1180 | FOREIGN LANGUAGE 2-1972 AMERICAN AIRLINE STEWARDESSES |
| 1181 | FOREIGN LANGUAGE 3-1972 AMERICAN AIRLINE STEWARDESSES |
| 1182 | POST HS EDUCATION 1-1972 AMERICAN AIRLINE STEWARDESSES |
| 1183 | POST HS EDUCATION 2-1972 AMERICAN AIRLINE STEWARDESSES |
| 1184 | COLLEGE EDUCATION-1972 AMERICAN AIRLINE STEWARDESSES |
| 1185 | RACE-1972 AMERICAN AIRLINE STEWARDESSES |
| 1186 | BRASSIERE SIZE-1972 AMERICAN AIRLINE STEWARDESSES |
| 1187 | CUP SIZE-1972 AMERICAN AIRLINE STEWARDESSES |
| 1188 | SIGNATURE-1972 AMERICAN AIRLINE STEWARDESSES |
| 1189 | ARM FOLDING-1972 AMERICAN AIRLINE STEWARDESSES |
| 1190 | HAND CLASPING-1972 AMERICAN AIRLINE STEWARDESSES |
| 1192 | STEWARDESS EXP-1972 AMERICAN AIRLINE STEWARDESSES |
| 1193 | LOCATION-1963 VIETNAMESE |
| 1194 | BIRTHPLACE-1963 VIETNAMESE |
| 1195 | ETHNIC DERIVATION-1963 VIETNAMESE |
| 1196 | HOME-1963 VIETNAMESE |
| 1197 | RELIGION-1963 VIETNAMESE |
| 1198 | SERVICE AND BRANCH-1963 VIETNAMESE |
| 1199 | RANK-1963 VIETNAMESE |
| 1200 | LENGTH OF SERVICE-1963 VIETNAMESE |
| 1201 | RANK-1959 ARMY AVIATORS |
| 1202 | BIRTHPLACE-1959 ARMY AVIATORS |
| 1203 | AERO DESIGNATION-1959 ARMY AVIATORS |
| 1204 | PARACHUTIST DESIGNATION-1959 ARMY AVIATORS |
| 1205 | QUALIFICATION-1959 ARMY AVIATORS |
| 1206 | HRS FLYING-FIXED-1959 ARMY AVIATORS |
| 1207 | HRS FLYING-ROTARY-1959 ARMY AVIATORS |
| 1208 | YRS SINCE RATED-1959 ARMY AVIATORS |
| 1209 | COMBAT FLYING-1959 ARMY AVIATORS |
| 1211 | BOOT SIZE (LNTH)-1966 NAVY PERSONNEL |
| 1212 | BOOT SIZE (WDTH)-1966 NAVY PERSONNEL |
| 1213 | RANK-1966 NAVY PERSONNEL |
| 1214 | PAY GRADE-1966 NAVY PERSONNEL |
| 1215 | LENGTH OF SERVICE-1966 NAVY PERSONNEL |
| 1216 | EDUCATION-1966 NAVY PERSONNEL |
| 1217 | MARITAL STATUS-1966 NAVY PERSONNEL |
| 1218 | BIRTHPLACE SUBJ-1966 NAVY PERSONNEL |
| 1219 | BIRTHPLACE FATHER-1966 NAVY PERSONNEL |
| 1220 | BIRTHPLACE MOTHER-1966 NAVY PERSONNEL |
| 1221 | LONGEST RESIDENCE-1966 NAVY PERSONNEL |
| 1222 | NATIONAL EXTRACTION-1966 NAVY PERSONNEL |
| 1223 | BIGGEST TOE-1966 NAVY PERSONNEL |
| 1224 | GLASSES-1966 NAVY PERSONNEL |
| 1225 | BIRTHPLACE SUBJ-1946 ARMY FEMALE SEPARATEES |
| 1226 | BIRTHPLACE FATHER-1946 ARMY FEMALE SEPARATEES |
| 1227 | BIRTHPLACE MOTHER-1946 ARMY FEMALE SEPARATEES |
| 1228 | EXTRACTION-1946 ARMY FEMALE SEPARATEES |
| 1229 | RELIGION-1946 ARMY FEMALE SEPARATEES |
| 1230 | EDUCATION-1946 ARMY FEMALE SEPARATEES |
| 1231 | OCCUPATION-1946 ARMY FEMALE SEPARATEES |
| 1232 | SPECIALTY-1946 ARMY FEMALE SEPARATEES |
| 1233 | MARITAL STATUS-1946 ARMY FEMALE SEPARATEES |
| 1234 | RACE-1946 ARMY FEMALE SEPARATEES |
| 1235 | CHILDREN-1946 ARMY FEMALE SEPARATEES |
| 1236 | MILITARY UNIT-1946 ARMY FEMALE SEPARATEES |
| 1237 | RANK-1946 ARMY FEMALE SEPARATEES |
| 1238 | LOCATION-1946 ARMY FEMALE SEPARATEES |
| 1239 | MONTH MEASURED-1988 ARMY PERSONNEL |
| 1240 | DAY MEASURED-1988 ARMY PERSONNEL |
| 1241 | YEAR MEASURED-1988 ARMY PERSONNEL |
| 1242 | UNIT-1988 ARMY PERSONNEL |
| 1243 | WAS SUBJ MEASURED-1988 ARMY PERSONNEL |
| 1242 | GENDER-1988 ARMY PERSONNEL |
| 1243 | MILITARY COMPONENT-1988 ARMY PERSONNEL |
| 1244 | MILITARY PERSONNEL CLASS-1988 ARMY PERSONNEL |
| 1245 | HANDEDNESS,WEAPON-1988 ARMY PERSONNEL |
| 1246 | WEAPON SITTING-1988 ARMY PERSONNEL |
| 1247 | HANDEDNESS,WRITING-1988 ARMY PERSONNEL |
| 1249 | RANK-1988 ARMY PERSONNEL |
| 1250 | GRADE-1988 ARMY PERSONNEL |
| 1251 | YEARS SERVED-1988 ARMY PERSONNEL |
| 1252 | MONTHS SERVED-1988 ARMY PERSONNEL |
| 1253 | AGE-1988 ARMY PERSONNEL |
| 1254 | BIRTH MONTH-1988 ARMY PERSONNEL |
| 1255 | BIRTH DAY-1988 ARMY PERSONNEL |
| 1256 | BIRTH YEAR-1988 ARMY PERSONNEL |
| 1257 | ESTIMATED HT, FEET-1988 ARMY PERSONNEL |
| 1258 | ESTIMATED HT, INCHES-1988 ARMY PERSONNEL |
| 1259 | ESTIMATED WEIGHT, LBS-1988 ARMY PERSONNEL |
| 1260 | RACE OF SUBJECT-1988 ARMY PERSONNEL |
| 1261 | RACE OF MOTHER-1988 ARMY PERSONNEL |
| 1262 | RACE OF FATHER-1988 ARMY PERSONNEL |
| 1263 | ETHNICITY OF SUBJECT-1988 ARMY PERSONNEL |
| 1264 | ETHNICITY OF MOTHER-1988 ARMY PERSONNEL |
| 1265 | ETHNICITY OF FATHER-1988 ARMY PERSONNEL |
| 1266 | BIRTH PLACE OF SUBJECT-1988 ARMY PERSONNEL |
| 1267 | BIRTH PLACE OF MOTHER-1988 ARMY PERSONNEL |
| 1268 | BIRTH PLACE OF FATHER-1988 ARMY PERSONNEL |
| 1269 | PRIMARY MOS (APHANUMERIC)-1988 ARMY PERSONNEL |
| 1270 | SECONDARY MOS (ALPHANUMERIC)-1988 ARMY PERSONNEL |
| 1271 | RANK-1950 AIR FORCE FLYERS |
| 1272 | AERORATING-1950 AIR FORCE FLYERS |
| 1273 | BIRTHPLACE-SUBJECT-1950 AIR FORCE FLYERS |
| 1274 | BIRTHPLACE-FATHER-1950 AIR FORCE FLYERS |
| 1275 | BIRTHPLACE-MOTHER-1950 AIR FORCE FLYERS |
| 1276 | RELIGION-1950 AIR FORCE FLYERS |
| 1277 | EDUCATION-1950 AIR FORCE FLYERS |
| 1278 | MARITAL STATUS-1950 AIR FORCE FLYERS |
| 1279 | HAIR COLOR-1950 AIR FORCE FLYERS |
| 1280 | HAIR FORM-1950 AIR FORCE FLYERS |
| 1281 | SKIN COLOR-1950 AIR FORCE FLYERS |
| 1282 | BODY HAIR QUANTITY-1950 AIR FORCE FLYERS |
| 1283 | BIRTHPLACE STATE-SUBJECT-1950 AIR FORCE FLYERS |
| 1284 1285 | BIRTHPLACE STATE-FATHER-1950 AIR FORCE FLYERS BIRTHPLACE STATE-MOTHER-1950 AIR FORCE FLYERS |
| 1286 | RANK - 1965 AIR FORCE MALE PERSONNEL |
| 1287 | AERORATING - 1965 AIR FORCE MALE PERSONNEL |
| 1288 | BIRTHPLACE - SUBJECT - 1965 AIR FORCE MALE PERSONNEL |
| 1289 | BIRTHPLACE - FATHER - 1965 AIR FORCE MALE PERSONNEL |
| 1290 | BIRTHPLACE - MOTHER - 1965 AIR FORCE MALE PERSONNEL |
| 1291 | RELIGION - 1965 AIR FORCE MALE PERSONNEL |
| 1292 | EDUCATION - 1965 AIR FORCE MALE PERSONNEL |
| 1293 | RACE - 1965 AIR FORCE MALE PERSONNEL |
| 1294 | MARITAL STATUS - 1965 AIR FORCE MALE PERSONNEL |
| 1295 | YEAR ENTERED AIR FORCE - 1965 AIR FORCE MALE PERSONNEL |
| 1296 | YEAR OF BIRTH - 1965 AIR FORCE MALE PERSONNEL |
| 1297 | YEAR MEASURED - 1965 AIR FORCE MALE PERSONNEL |
| 1299 | AFSC - 1968 SURVEY OF ARMY WOMEN |
| 1300 | RACE - 1968 SURVEY OF ARMY WOMEN |
| 1301 | MARITAL STATUS - 1968 SURVEY OF ARMY WOMEN |
| 1302 | RANK - 1968 SURVEY OF ARMY WOMEN |
| 1303 | COMMAND - 1968 SURVEY OF ARMY WOMEN |
| 1304 | LOCATION - 1968 SURVEY OF ARMY WOMEN |
| 1305 | BIRTHPLACE - SUBJECT - 1968 SURVEY OF ARMY WOMEN |
| 1306 | BIRTHPLACE - FATHER - 1968 SURVEY OF ARMY WOMEN |
| 1307 | BIRTHPLACE - MOTHER - 1968 SURVEY OF ARMY WOMEN |
| 1308 | YEAR OF BIRTH - 1968 SURVEY OF ARMY WOMEN |
| 1309 | AGE AT MENARCHE - 1968 SURVEY OF ARMY WOMEN |
| 1310 | YEAR MEASURED - 1968 SURVEY OF ARMY WOMEN |

# HSIAC Anthropometric Data Archive Survey List

U.S. Military

## Air Force

1990 Air Force Male Flyers

1968 Survey of Air Force Women

1967 Survey of Flying Personnel – Male

1965 USAF Male Personnel

1950 Survey of USAF Flying Personnel

## Army

1988 Anthropometric Survey of the U.S. Army Female Working Data Set

1988 Anthropometric Survey of the U.S. Army Male Working Data Set

1970 Army Aviators

1959 Survey of Army Aviators

1946 Survey of Army Female Separatees

## Navy/Marines

1966 Survey of Marine Personnel

1966 Survey of Navy Personnel

Other Military

1976 Anthropometric Survey of Royal Transport Corpsmen

1975 Survey of English Guardsmen

1974 Canadian Forces Survey

1972-1975 British Army

1972 Survey of 500 Royal Air Force Aircrew Heads

1971 Survey of Royal Air Force Crewmen

1971 Royal Australian Air Force

1969 Imperial Iranian Armed Forces

1968 German Air Force

1963 Vietnam Military

1960-61 NATO Anthropometric Survey

1959 Survey of Latin American Military

Civilian

1975 Law Enforcement Officers

1972 American Airline Stewardesses

1960-62 The Health Examination Survey

1961 Air Traffic Controllers